|  | Worksheet 1 Simple DC motor control CP0507-1 |  | Motors and microcontrollers |
| --- | --- | --- | --- |
|  | There is a huge number of devices that use small motors including toys, electric toothbrushes, medical and mechatronics systems. Turning them on and off requires the use of a relay or a transistor. Varying the speed requires the use of Pulse Width Modulation as a power control technique. In this worksheet you will learn how to use a potentiometer and PWM to control the speed of a simple DC motor. |  |  |

https://youtu.be/e4gB8YcOp8I

Over to you:

- Watch the video “Controlling DC motors – Simple DC” on the Flowcode YouTube site.

- Load the file “Simple DC". Set up the hardware appropriately.

- The program allows you to control the speed of the motor using a potentiometer.

Challenge:

- Modify the program so that two switches control the speed of the motor: plus and minus.

- Print the speed on the LCD.

Hints:

In the main loop detect if a switch has been pressed – say switch PORTA0 and PORTA1 on a combo board or switch board on port A.

Use a variable SPEED and add or subtract 1 from SPEED as the appropriate switch is detected.

Use two IF icons for the logic – IF switch A0 is pressed or IF switch A1 is pressed

Put the appropriate logic in the YES branch of the IF icons

Replace the two IF icons with a SWITCH icon.