|  | worksheet> <filename> “CP0507-3.doc” </filename> Worksheet 3 Servo motor control |  | Motors and microcontrollers |
| --- | --- | --- | --- |
|  | Small servo motors use a miniature DC motor with positional feedback control to allow you to control the angle of a motor rather than the speed. The angle is changed to a linear position control by using a small arm on the rotor with a rod. This allows the flaps on radio-controlled aeroplanes to be controlled remotely. In this worksheet you learn how to control simple servo motors. |  |  |

https://youtu.be/U-GoNwE5kPk

## Over to you:

Watch the video “Controlling Servo motors” on the Flowcode YouTube site.

Load the file “Servo". Set up the hardware appropriately.

The program uses a potentiometer to adjust the position of the servo motor.

## Challenge:

Use a logic analyser to view the waveform generated by the microcontroller for different positions of the servo motor.

Calibrate the motor angle for 0 and 255 full scale.

Use some maths to display the angle on the LCD.

## Hints:

- Set up a Real Type variable – Angle.

Make sure you use the decimal point on all Real calculations: e.g. “2.0” not “2”.

Measure the angle (visually) for an output of 0 and 255.

Calculate the angle and display it on the LCD.