|  | Worksheet 2 Light sensor CP1972-2 |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | The output voltage from a light sensor either gets higher with more light, or lower. This allows us to detect the light level so that we know when to turn street lights on and off. In this example you investigate the light level and make an adjustable light switch. |  |  |

There is no video for this example.

Over to you:

- Load the file “Light sensor". Set up the hardware appropriately.

- The program reads the value of the light sensor and displays it as a number on the LCD.

- Shine a mobile phone light or a torch on the sensor to establish what the numerical range of the sensors is.

Challenge:

- Make an electronic light switch that turns a LED off when the light level goes high.

- Add a potentiometer to your program which adjusts the light level at which the switch is made.

Hints:

- Add a variable LightLevel and read the light sensor value into this variable

In the Main loop use an IF icon to compare LightLevel to the light sensor input. Alter the LED output accordingly.

Use two lines on the LCD display to show light input level, LightLevel, and to make sure your program is working.