|  | Worksheet 2 Full bridge motor control CP0507-2 |  | Motors and microcontrollers |
| --- | --- | --- | --- |
|  | Sometimes we need to control the direction of a motor – for example an electric wheelchair. Wheelchairs typically make use of a pair of 12 or 24V DC motors – one on each drive wheel. A microcontroller /joystick system allows users to go forwards, backwards and also turn left and right. A full bridge circuit – typically in a single chip/module these days - provides this control. In this worksheet you learn how to control a motor using a full bridge system. |  |  |

https://youtu.be/-ajj-YChfro

Over to you:

- Watch the video “Controlling DC motors – full bridge” on the Flowcode YouTube site.

- Load the file “Full bridge". Set up the hardware appropriately.

- The program cycles the motor forwards and backwards at a fixed speed.

Challenge:

- Modify the program so that two switches control the direction of the motor: forwards and backwards and two switches control the speed.

- Print speed and the direction on the LCD.

Hints:

- In the main loop detect if a switch has been pressed – say A0, A1, A2, A3

Use a SWITCH icon for the logic – assume only one switch is pressed at any time

Put the appropriate logic in the branches of the SWITCH icon.