CP4807

Introduction to microcontrollers

|  | Preparation | Introduction to microcontrollers |
| --- | --- | --- |

Before tackling the worksheets you will need to understand some basic information:

### Microcontroller basics

This document gives a basic explanation as to how microcontrollers work.

### Introduction to E-blocks

This document explains how the E-blocks system works.

### E-blocks datasheet

This document gives the full schematics of all the E-blocks 3 circuit boards.

### Flowcode Embedded getting started guide

This document shows you how to construct your first program from scratch.

### Introduction to Flowcode Embedded video

https://youtu.be/_9WAcrK9LjE

|  | Worksheet 1 First program CP4807-1 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | This first program introduces you to several programming commands: OUTPUT, LOOP and DELAY. In this first program you explore how to get a single output on the microcontroller turning on and off using these commands. |  |  |

https://youtu.be/h8-7BsXBpLc

Over to you:

- Watch the video “Flowcode Embedded: first program” on the Flowcode YouTube site.

- Load the file “First program". Set up the hardware appropriately, download it, and make sure you understand it.

- The program flashes a single LED on the combo board.

Challenges:

- Delay: alter the delay period. Make sure you understand how the Delay command works.

- Output: change the output to work with port A and port B

- Output: Select Advanced options and alter the bit that is flashed

- Loop: experiment with the loop dialogue box. Flash the LED for only 10 times.

- Alter the program so that it flashes Port 1 bit 0 for 10 times, then Port 2 bit 1 for 9 times then port 2 bit 2 for 8 times.

Hints:

You can use three loops one after the other

Alter the Loop count in each one

Use the Advanced options in the Output icon to control which output bit is used.

|  | Worksheet 2 Performing calculations CP4807-2 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Microcontroller brains are not wired like human being’s. This has consequences for mathematics, variables and input output processes that you need to understand. In this worksheet you explore calculations and binary and hexadecimal numbering systems. |  |  |

https://youtu.be/iEkocOsMj6Q

Over to you:

- Watch the video “Binary, Hex and Magic numbers” on the Flowcode YouTube site.

- Load the file “B H M N". This is a simulation only program.

- There are some commands in here that you will not yet understand. Don’t worry about that for now: just look at the results of the simulation.

Challenges:

- Simulate the program and pause it or slow it down to help you understand.

- Make sure you understand conversion between binary, Hex and Decimal.

- Test yourself by selecting 3 decimal numbers between 0 and 100 and converting them to binary

- Repeat the exercise but convert 3 binary 8 bit numbers into decimal.

If you are using an ESP32 microcontroller this worksheet will present difficulties as the ESP32 is a 32 bit system and chip input output connections are complex.

https://youtu.be/T7VX-QNLzeQ

Over to you:

- Watch the video “Flowcode Embedded: a counter” on the Flowcode YouTube site.

- Load the file “Counter". Set up the hardware appropriately, download it, and make sure you understand it.

- The program counts in binary on a port of the microcontroller

Challenges:

- Slow down the program by altering a delay. Make sure you understand the counting sequence.

- In the Project Manager click on the variable Count to see the Variable dialogue control. Count is Type Byte. Investigate the Types Bool, Int and Uint.

- Alter the program so that initially Count = 1. Alter the calculation in the Loop so that Count = Count * 2 rather than Count = Count +1. Can you explain what happens when you run the program?

- Alter the program so that after the 7th LED is lit up the program then lights bit 6, then bit 5 etc.

Hints:

- The Flowcode Wiki contains information on variable Types

Use the Loop count so that the first loop executes 7 times. At this point bit 7 is lit up. Add a second loop that executes 7 times but use the divide ‘/’ operator in the calculation command rather than the times ‘*’ operator.

|  | Worksheet 3 Using connection points CP4807-3 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | There are always multiple ways of making a program. Sometimes this is down to personal preference, sometimes this is down to good programming technique. In this exercise you explore two ways of making a program: using connection points and loops. |  |  |

There is no video for this worksheet.

Over to you:

- Load the results of the previous worksheet. You should have a program that lights up the LEDs on a port in sequence going from bit 0 to 7 and then bit 7 to 0.

- Use connection points to make the program run forever.

Challenges:

- Using connection points start from scratch and create a traffic light program that forever loops between ‘Red’ for 10 seconds, ‘Amber’ for one second, ‘Green’ for 10 seconds, ‘Amber’ for 1 second. ‘Red’ for 10 seconds and so on.

- Repeat the exercise without using any connection points – just use Loops.

- Which is the easiest program to understand?

Hints:

- Add a connection point at the start of the program.

- At the end of the program add a Goto Connection point.

- Traffic lights: use a single loop with Output commands and delays to achieve the result.

|  | Worksheet 4 Digital inputs CP4807-4 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | We need a way of controlling electronic devices locally. Often this is done in a digital context – with different types of switches. In this case we need to get the input ‘commands’ and then process this to alter the state of the microcontroller or alter its processes. In this exercise you look at how to get command data into a microcontroller. |  |  |

Over to you:

- Watch the video “Flowcode Embedded: digital inputs” on the Flowcode YouTube site.

- Load the file “Digital inputs". Set up the hardware appropriately, download it, and make sure you understand it.

- The program reflects the input on one 8 bit port to another by storing the port status into a variable and then sending that to the output of another port

Challenges:

- Alter the program so that it works on only the least significant bit using a Boolean variable.

- Alter the program so that the state of the Least significant bit input is reflected to the most significant bit of the same port.

Hints:

- Setup a new variable called ‘LSBinput’ of Type Boolean.

- Use the Advanced options of the Input command to read bit 0 into LSBinput.

- Change the Output icon using Advanced options so that it outputs LSBinput to bit 7 of the same port as the input.

|  | Worksheet 5 Making decisions CP4807-5 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | The behaviour of a program and its processes will often depend on the choices made by the user via a control panel or input from sensors. The way we make decisions in a program is through the IF statement and the Switch statement. In this worksheet you explore how decisions are made in microcontroller programs. |  |  |

Over to you:

- Watch the video “Flowcode Embedded: making decisions” on the Flowcode YouTube site.

- Load the file “Making decisions". Set up the hardware appropriately, download it, and make sure you understand it.

Challenges:

- Alter the program so that when Switch A.0 and switch A.4 are pressed all the LEDs on port B are lit.

- Use a Switch command icon with 8 branches so that when the switch on A.0 is pressed the LED on B.7 is lit, when switch A.1 is pressed the LED on B.6 is lit etc.

Hints:

- Use your knowledge of binary to work out the input when A0 AND A4 are pressed.

- Write down the binary input for each switch press

- Set up a Switch statement with 8 branches which tests for each switch press

|  | Worksheet 6 Macros / subroutines CP4807-6 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | In designing microcontroller systems we use code that is very repetitive and we can simplify the development by using software macros or subroutines. Pre written libraries of code for hardware modules also save us a lot of time, and help us create more functional systems. In this worksheet you explore the use of hardware and software macros. |  |  |

### https://youtu.be/hNBSxdb72v4

Over to you:

- Watch the video “Macros” on the Flowcode YouTube site.

- Load the file “how to use macros". Set up the hardware appropriately, download it, and make sure you understand it.

Challenges:

- Alter the program so that it Prints “Count is: “ then the count number

- Alter the program so that it prints the number on the second line of the display rather than the first

- Assign the text “Count is: “ to a variable and use the string variable in your program.

- Add a second software macro to the program called “Initialise”.

- Move the Start command to this macro and then call it from the Main routine.

Hints:

- Use the hardware macro PrintString to print a string (you need to use inverted commas in the dialogue box)

- Add a new variable “mystring” of Type String and length 20. Use PrintString to print this.

- Use the Cursor hardware macro to alter the print position.

|  | Worksheet 7 Colour touch screen CP4807-7 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Graphical displays extend the functionality of electronic systems, allow more information about be displayed, and allow information to be displayed in more easily understood formats: tables, graphics, graphs etc. Graphical display touch screens are also replacing LEDs, LCD displays and switches and potentiometers as input devices. In this worksheet you explore how graphical colour displays work. |  |  |

### https://youtu.be/xxxxxxxx

Over to you:

- Watch the video “Colour touch screen” on the Flowcode YouTube site.

- Load the program “Colour touch screen”. Set up the hardware appropriately, download it, and make sure you understand it.

Challenges:

- Set up 4 touch screen buttons at the top of the screen

- Set up 2 virtual LEDs below the first two buttons

- The first two buttons are switches. Create a program that switches the two virtual LEDs on and off. Mirror the status to the first two LEDs on Port A on the Combo board.

- The second two buttons are “+” and “-“ buttons. Create a variable “Value” type Byte. Write a program that alters Value up and down when each button is pressed. Display Value on the screen below the buttons to the right of the virtual LEDs.

Hints:

- Plan your project first. This will save you time. Draw on paper the position of all of the assets. The screen is 480 x 320 pixels.

- Create the graphics and text areas first. Simulate to verify.

- Modify the program so that it detects the 4 button areas and a screen press. You can just use 4 IF statements one after the other. In the YES branch of each statement put the logic you want for when that button is pressed.

- You may get need to alter the delay at the end of the while loop to ’debounce’ your button press.

Over to you:

- Debouncing is a problem with touch screens. In practice you need to track if there is a change in the status of the touch screen and then only act on it. If you have time write a program that solves this problem.

Hints:

- Use OLD and NEW variables to track the status of the touch and only make changes in status when there is a valid change.

|  | Worksheet 8 Pin interrupts CP4807-8 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | In some systems there are possible events that are really important – like the kill switch on a factory machine. When switches like this are activated you need your program to stop whatever it is doing – ‘interrupt’ the program - and take action. Interrupts allow us to do this. In this worksheet you explore how pin interrupts work. |  |  |

### https://youtu.be/imTq19jX408

Over to you:

- Watch the video “Flowcode Embedded: pin interrupts” on the Flowcode YouTube site.

- Load the file “pin interrupts". Set up the hardware appropriately, download it, and make sure you understand it.

Challenges:

- Alter the program so that a LED output flashes during normal operation.

- Alter the program so that once the ‘stop switch’ on the interrupt pin is activated the program waits until a ‘clear’ switch is pushed before continuing the program.

Hints:

- Put a flashing LED routine in the main loop.

- In the Interrupt routine insert a While…until loop.

- Use a variable to track the status of the ‘clear’ switch.

- In the loop test for the ‘clear’ switch being pressed.

|  | Worksheet 9 Timer interrupts CP4807-9 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Many microcontroller systems need some kind of timing system. Microcontrollers are designed to help us here as they have dedicated internal hardware that counts the number of internal clock pulses and converts them into time. When activated these internal counters can ‘interrupt’ the program to help you manage time-based code. In this worksheet you explore how timer interrupts work. |  |  |

https://youtu.be/E5SWKxIjvno

Over to you:

- Watch the video “Flowcode Embedded: timer interrupts” on the Flowcode YouTube site.

- Load the file “timer interrupts". Set up the hardware appropriately.

Challenges:

- Alter the program so that it calculates the hours, minutes and seconds that have elapsed.

- Use a switch and a pin interrupt to clear the timer.

Hints:

- Create variables hour_count and minute_count

- In the main loop use IF statements to track when second_count is more then 60 and reset it to 0 and increment minute_count Similarly for hour_count.

- Keep the programming in the interrupt routine to a minimum – its good practice

- Create a new routine for a pin interrupt and use the code from the previous exercise to create the reset function.

|  | Worksheet 10 Touch menu systems CP4807-10 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Touchscreen systems are replacing the use of switches, potentiometers, LEDs and other electronic devices. Using a graphical touch screen reduces costs and enhances functionality. In this worksheet you learn how to develop a more complex touch screen system |  |  |

Scene 1Display screen

Program should have at least 3 Scenes

Navigation between scenes

Temperature sensor

Heater output

- Display screen

- Set up screen – temperature of switch on

- Timing screen – maybe?

|  | Teacher’s notes | Introduction to microcontrollers |
| --- | --- | --- |

## Previous learning

Some understanding of electronics is needed.

## Pedagogy

This set of worksheets provides structure for the study of microcontroller programming. The worksheets provide structure to the learning but do not provide all the information students need. Our vision as to how you teach this subject is:

- This is a very practical subject. We expect that 90% of the time students spend on this module will be spent working alone.

- Each student should have Flowcode Embedded software and a set of E-blocks 3 hardware. Don’t be tempted to try to get students to learn this subject in pairs.

- Students should be  introduced to each topic or worksheet. They should be given time to investigate the topic using the internet. If students are investigating this on their own you may want to give additional structure to this activity.

- Students should then load an example program and download this to the microcontroller so that they can see how the program works. Example programs are available on the Flowcode Wiki for each of PIC, Arduino Mega and ESP32 microcontrollers.

- Students should be given the opportunity to watch the accompanying YouTube video – individually or as a class activity. This explains the program.

- Students then modify each example as detailed in the worksheet. This ensures that they have success, but also ensures that they have understanding.

- Occasionally you should test students understanding by asking them to complete a project from scratch. This tests their understanding of what they have learned.

- Students do not have to do all worksheets or tasks. That’s up to you to decide.

## Example programs

The Flowcode wiki includes basic examples for each worksheet. Completed programs for the worksheet tasks are not given. Students are expected to construct the programs themselves.

## Time

Of course this depends on student capability. We estimate that it will take 15 hours to complete all the worksheets.

## Software

The videos make use of Flowcode embedded and we anticipate that you are likely to use this for your programs. But you can use these worksheets with any microcontroller development software. The flowchart programs can easily be followed by C programmers.

## Assessment

Creating any of these programs from scratch will be suitable as a way of assessing that students have understood the programs and have programming ability.

## Hardware

This course is designed to be used with a Matrix Microcontroller development centre. There are three versions:

BL5394		ESP32 microcontroller development centre

BL8624		PIC microcontroller development centre

BL3797		Arduino microcontroller development centre

| Version control | Introduction to microcontrollers |
| --- | --- |

28 05 25	First beta