CO0002 content

<HTML> Homework 1 <filename> “CO0002-H1.htm” </filename>

Homework 1

First program

Performing calculations

Digital inputs

Using connection points

Make sure that you can replicate the examples from scratch. This make sure that you understand them.

Design brief:

Design a program that controls two sets of traffic lights at a junction.

Hardware:

Upstream board

Use a Combo board on port A and Port B where the connections to the traffic lights are represented by:

Port A bit 0 GREEN

Port A bit 1 AMBER

Port A bit 2 RED

Port B bit 0 GREEN

Port B bit 1 AMBER

Port B bit 2 RED

Software

Create a panel in Flowcode with the two sets of lights in the appropriate colour. You can do this using the single LED component. You can carry out this program in simulation and then try it on the hardware next time you are in class.

Design the program so that the lights at the junction control the traffic in an appropriate sequence of:

RED: 20 seconds

AMBER: 3 seconds

GREEN: 20 seconds

When there is RED at one set of lights there must be GREEN at the other.

</HTML>

<HTML> Homework 2 <filename> “CO0002-H2.htm” </filename>

Homework 2

Making decisions

Macros and subroutines

Colour graphical displays

Design brief:

Carry on with the worksheet on Colour Graphical displays.

Hardware:

Upstream board

Analogue Sensor board with potentiometer on the appropriate port (Port F for Arduino and any port for PIC and ESP)

Switch board on port A

Software

You should have created a panel with 8 switches and a potentiometer. You can carry out this program in simulation and then try it on the hardware next time you are in class.

</HTML>

Homework 3

<HTML> Homework 3 <filename> “CO0002-H3.htm” </filename>

Colour graphical displays

Analogue inputs

Light sensor

Design brief:

Design a program that controls two sets of traffic lights at a junction.

Hardware:

Upstream board

Analogue Sensor board with light sensor on the appropriate port (Port F for Arduino and any port for PIC and ESP)

Software

Create a program that reads in the value of the light sensor.

Dedicate an area of the Colour display to be a ‘light’.

Draw a colour rectangle in the dedicated area which is inversely proportional to the value from the light sensor. The colour of the rectangle is dictated by R, G, B values and you can use these to dictate the colour.

Add text to your display to give your project a name.

</HTML>

Homework 4

<HTML> Homework 4 <filename> “CO0002-H4.htm” </filename>

Digital accelerometer

Design brief:

Carry on with the worksheet on the digital level.

Hardware:

Upstream board

Accelerometer sensor

Software

Show the level with a display on the bottom of the screen – use a moving circle to show the level. The circle will be in the centre of the display when level.

On the right of the screen show the vertical component –the circle will be in the centre when Z = 0.

</HTML>

Homework 5

<HTML> Homework 5 <filename> “CO0002-H5.htm” </filename>

Work on Assessment 2

</HTML>

Homework 6

<HTML> Homework 6 <filename> “CO0002-H6.htm” </filename>

DC motor control

Full bridge control

Design brief:

Create a program that uses a potentiometer to control the speed of the motor.

Hardware:

Upstream board

Combo board or switch board

Actuators board

Software

Use two switches to control whether the motor turns forwards or backwards. The switches are push to make, so you will need to use logic to detect a switch is pressed and then set a DIRECTION flag. Use the Colour graphical display to show the user what is happening in the program.

</HTML>

Homework 7

<HTML> Homework  7<filename> “CO0002-H7.htm” </filename>

Timer interrupts

Design brief:

Create a stopwatch program.

Hardware:

Upstream board

Combo board or switch board

Software

Create a stopwatch program with the following features:

A single start/stop switch

A reset switch

Colour graphical display with the time in 7-segment font in the centre of the display

Display nicely formatted with title and well designed.

</HTML>

Homework 8

<HTML> Homework  8<filename> “CO0002-H8.htm” </filename>

Touch control systems

Design brief:

Carry on with the worksheet on the touch screen project.

Hardware:

Upstream board

Combo board

Software

As dictated in the Challenge section in the worksheet.

</HTML>

Homework Homework  9

<HTML> <filename> “CO0002-H9.htm” </filename>

Students need to plan their project.

The plan should have the following sections:

Design brief:

A full design brief with sketches of the panel, the hardware, and a sketch of the Colour touch screen display graphics if this is used. Planning is important.

Hardware:

A description of the hardware used. E-blocks boards used and any prototype board circuitry.

Software

A description of the software functionality. Partly described by the Colour touch screen sketches.

Technical risk

A list of the technical risks in the project – what does the students not know? What are the areas of difficulty? How will the student overcome these difficulties? What does the student need to learn.

</HTML>

<HTML> Assessment 1<filename> “CO0002-A1.htm” </filename>

Assessment 1

This first assessment is designed to test students on what they have learned so far. The project is one that they have carried out before – a simple light sensor and a section on the colour display that acts as a ‘light’.

Students should start the program from scratch rather than loading previous work. This will require some supervision. IF students have done this work then this will reinforce their understanding and it will giver supervisors a good idea of which students are on top of the subject and which are not.

Design brief:

Design a program that controls two sets of traffic lights at a junction.

Hardware:

Upstream board

Analogue Sensor board with light sensor on the appropriate port (Port F for Arduino and any port for PIC and ESP)

Software

Create a program that reads in the value of the light sensor.

Dedicate an area of the Colour display to be a ‘light’.

Draw a colour rectangle in the dedicated area which is inversely proportional to the value from the light sensor. The colour of the rectangle is dictated by R, G, B values and you can use these to dictate the colour.

Add text to your display to give your project a name.

</HTML>

Assessment 2

<HTML> Assessment 2 <filename> “CO0002-A2.htm” </filename>

This is part in classroom and part homework. A 6 hour block  of time that acts as a mini project.

Design brief:

Create a digital temperature controller that allows you to control the temperature in a room.

Hardware:

Upstream board

Combo board or switch board

Digital temperature sensor board

Relay board

Software

Design the Colour graphical display with large characters for the current temperature as measured by the sensor board.

Below the main display have a second text field with the target temperature.

Use two switches, up and down, to control the target temperature value and display. Update the target temperature as the switches are pressed.

When the actual temperature is greater than the target temperature the heater is OFF. When the actual temperature is less than the target temperature the heater is ON.

Use a text field on the top right of the display to show the heater status: ON or OFF.

Use a single LED to display the heater status.

Activate the relay when the heater should be on.

</HTML>

<HTML> Assessment 3 <filename> “CO0002-A3.htm” </filename>

Assessment 3

This is part in classroom and part homework. A 6 hour block of time that acts as a mini project.

Design brief:

Enhance the digital temperature controller with touch functionality.

Hardware:

Upstream board

Combo board or switch board

Digital temperature sensor board

Relay board

Software

Design the Colour graphical display with large characters for the current temperature as measured by the sensor board.

Below the main display have a second text field with the target temperature.

Use two virtual switches on the touch screen for UP and DOWN.

When the actual temperature is greater than the target temperature the heater is OFF. When the actual temperature is less than the target temperature the heater is ON.

Use a text field on the top right of the display to show the heater status: ON or OFF.

Use a single LED to display the heater status.

Activate the relay when the heater should be on.

Add a third touchable area on the display that acts as an Over ride button to turn the heater on for 10 minutes even when the actual temperature is above the set temperature.

Add a field to the display that shows the time using a timer interrupt.

</HTML>

<HTML> Project <filename> “CO0002-P1.htm” </filename>

Project

Students should develop a project of their own which proves their capability. They should discuss this with the teacher.

Design brief:

A full design brief with sketches of the panel, the hardware, and a sketch of the Colour touch screen display graphics if this is used. Planning is important.

Hardware:

A description of the hardware used. E-blocks boards used and any prototype board circuitry.

Software

A description of the software functionality. Partly described by the Colour touch screen sketches.

Technical risk

A list of the technical risks in the project – what does the students not know? What are the areas of difficulty? How will the student overcome these difficulties? What does the student need to learn.

</HTML>

<HTML> Assessment and marking scheme <filename> “CO0002-marking scheme.htm” </filename>

Assessment marking schemes

Bronze (Functional & Basic): The project works but may be brittle, poorly documented, or lack error handling. It meets minimum requirements.

Silver (Robust & Structured): The code is clean, well-structured, follows standards, and includes documentation/tests. It is "developer-ready".

Gold (Polished & Production-Ready): The project is optimized, highly scalable, and user-ready. It includes comprehensive testing, documentation, and handles edge cases gracefully. It exceeds the design brief and shows real design flair.

</HTML>

<HTML> Welcome <filename> “CO0002 – welcome.HTM””</filename>

Welcome to the Introduction to Microcontrollers course.

More here about what you will learn and how you will learn it.

</HTML>

<HTML> Learning objectives <filename> “CO0002 – LO.HTM””</filename>

Within this course the learning objectives are closely aligned to the requirements of Unit 6 of the BTEC National. These are all included in this course. They are:

| A1 Control hardware |  |
| --- | --- |
| I/O capabilities – number, type (analogue/digital), ports |  |
| hardware specification – bus width, processor speed |  |
| memory – RAM, ROM |  |
| hardware features – interrupts, stack, PWM |  |
| required peripherals |  |
| cost and accessibility |  |
| ease of use |  |
| software and programming language |  |
| operating voltages and power requirements |  |
| A2 Input devices |  |
| User input: |  |
|  | digital – switches and buttons |
|  | analogue – control potentiometer |
| Temperature |  |
|  | Thermistor |
|  | temperature sensors |
|  | environmental sensor – temperature and humidity |
| Light |  |
|  | light-dependent resistor (LDR) |
|  | IR – phototransistor, photodiode or IR receiver |
| Movement/orientation |  |
|  | tilt switch |
| Presence |  |
|  | micro-switch |
|  | Ultrasonic |
| Input interfacing requirements |  |
|  | signal conditioning |
|  | analogue-to-digital (ADC) conversion |
|  | modular sensor boards |
|  | PWM |
|  | serial communications |
|  | Inter-Integrated Circuit (I2C) |
| A3 Output devices |  |
| Optoelectronic |  |
|  | light-emitting diode (LED) – indicator and IR |
|  | 7-segment display |
|  | liquid crystal display (LCD) |
| Electromechanical |  |
|  | Relay |
|  | direct current motor |
|  | Servo |
| Audio |  |
|  | buzzer or siren |
|  | speaker or piezo transducer |
| Output interfacing requirements |  |
|  | power requirements and drivers |
|  | transistor output stage |
|  | Relay |
|  | PWM |
|  | serial communications |
|  | I2C device interfacing |
| A4 Selecting hardware devices and system design |  |
| A5 Assembling and operating a microcontroller system |  |
| A Investigate typical microcontroller system hardware |  |
| A1 Control hardware |  |
| I/O capabilities – number, type (analogue/digital), ports |  |
| hardware specification – bus width, processor speed |  |
| memory – RAM, ROM |  |
| hardware features – interrupts, stack, PWM |  |
| required peripherals |  |
| cost and accessibility |  |
| ease of use |  |
| software and programming language |  |
| operating voltages and power requirements |  |
| A2 Input devices |  |
| User input: |  |
|  | digital – switches and buttons |
|  | analogue – control potentiometer |
| Temperature |  |
|  | Thermistor |
|  | temperature sensors |
|  | environmental sensor – temperature and humidity |
| Light |  |
|  | light-dependent resistor (LDR) |
|  | IR – phototransistor, photodiode or IR receiver |
| Movement/orientation |  |
|  | tilt switch |
| Presence |  |
|  | micro-switch |
|  | Ultrasonic |
| Input interfacing requirements |  |
|  | signal conditioning |
|  | analogue-to-digital (ADC) conversion |
|  | modular sensor boards |
|  | PWM |
|  | serial communications |
|  | Inter-Integrated Circuit (I2C) |
| A3 Output devices |  |
| Optoelectronic |  |
|  | light-emitting diode (LED) – indicator and IR |
|  | 7-segment display |
|  | liquid crystal display (LCD) |
| Electromechanical |  |
|  | Relay |
|  | direct current motor |
|  | Servo |
| Audio |  |
|  | buzzer or siren |
|  | speaker or piezo transducer |
| Output interfacing requirements |  |
|  | power requirements and drivers |
|  | transistor output stage |
|  | Relay |
|  | PWM |
|  | serial communications |
|  | I2C device interfacing |

</HTML>

<HTML> Other resources <filename> “Other resources.HTM”</filename>

There are other places where you can look for help and support:

Flowcode has a dedicated web site with lots of information on it. You can see that here:

www.flowcode.co.uk

There are getting started guides for Flowcode and E-blocks which takes you step by step through your first program. You can see that here:

https://www.flowcode.co.uk/wiki/index.php?title=Embedded_Getting_Started_Guide

There is a fantastic Wiki with huge amounts of information and examples on Flowcode. You can see that here;

https://www.flowcode.co.uk/wiki/index.php?title=Main_Page

There is a great forum for Flowcode where you can post questions and get support. You can see that here:

https://www.flowcode.co.uk/forums/

spend a bit of time on each of these parts of the Flowcode web site and familiarise yourself with how they work.

</HTML>

<HTML> Now try the worksheets <filename> “Now try the worksheets.HTM”</filename>

In the last few screens you have learned about Flowcode and E-blocks. Hopefully you have a basic grasp of how the equipment works and where to look for more information.

The next step is that you need to try the equipment for yourself and understand how the worksheets structure the students’ learning and will help you learn how to use Flowcode and E-blocks.

In the following screens we will present you with the first 4 worksheets that students follow. We recommend that you take the time to go through them.

</HTML>

Scheme of work – 120 hour

The 120 hour scheme of work includes the 60 hour scheme of work and the following:

Allcode competition

Formula Allcode is a small mobile robot powered by a PIC microcontroller. Allcode is a fantastic tool for teaching Autonomous mobile robotics through a range of exercises including line following, tune playing and dancing and maze following.

PC interfacing

In this module students learn how to create apps for a PC which interface to low cost embedded systems like Arduino Uno.

Working with mechanical sensors

Working with strain sensors, Thermistors, pressure sensors, and power boards.

DC motor control

Students understand how to control DC motors using feedback.

Further project

Students undertake a larger project.

|  | Version control | Introduction to microcontrollers |
| --- | --- | --- |

04 12 25	First release

18 02 26	Second release with schemes of work

05 04 26 	with AI mark up instructions

10 04 26	More courses