<worksheet> <filename> “CP4807-head.doc” </filename>

Introduction to microcontrollers

CP4807

</worksheet>

|  | <worksheet> <filename> “CP4807-Cont.doc” </filename> Contents | Introduction to microcontrollers |
| --- | --- | --- |

Bronze

Worksheet 1 		First program

Worksheet 2		Performing calculations

Worksheet 3		Connection points

Worksheet 4		Digital inputs

Worksheet 5		Making decisions

Worksheet 6		Macros / subroutines

Worksheet 7		Using prototype boards

Silver

Worksheet 8		Colour graphical displays

Worksheet 9		Pin interrupts

Worksheet 10		Timer interrupts

Gold

Worksheet 11		Touch control systems

Worksheet 12		Web mirror

Teacher’s notes

</worksheet>

|  | <worksheet> <filename> “CP4807-0.doc” </filename> # Preparation | Introduction to microcontrollers |
| --- | --- | --- |

Before tackling the worksheets you will need to understand some basic information:

## Microcontroller basics

This document gives a basic explanation as to how microcontrollers work.

## Introduction to E-blocks

This document explains how the E-blocks system works.

## E-blocks datasheet

This document gives the full schematics of all the E-blocks 3 circuit boards.

## Flowcode Embedded getting started guide

This document shows you how to construct your first program from scratch.

Introduction to Flowcode Embedded video

https://youtu.be/_9WAcrK9LjE

## IMPORTANT

Before you start using Flowcode make sure that you have the latest libraries by selecting HELP…LIBRARY UPDATES…FULL DATABASE (FILES IN USE DROP BOX)…DOWNLOAD.

</worksheet>

|  | <worksheet> <filename> “CP4807-1.doc” </filename> Worksheet 1 First program |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  |  |  | <image>flashing light.jpg</image> |
|  | This first program introduces you to several programming commands: OUTPUT, LOOP and DELAY. In this first program you explore how to get a single output on the microcontroller turning on and off using these commands. |  |  |

https://youtu.be/h8-7BsXBpLc

## Over to you:

Watch the video “Flowcode Embedded: first program” on the Flowcode YouTube site.

Load the file “First program". Set up the hardware appropriately, download it, and make sure you understand it.

The program flashes a single LED on the combo board.

## Challenges:

Delay: alter the delay period. Make sure you understand how the Delay command works.

Output: change the output to work with port A and port B. Output: Select Advanced options and alter the bit that is flashed

Loop: experiment with the loop dialogue box. Flash the LED for only 10 times.

Alter the program so that it flashes Port 1 bit 0 for 10 times, then Port 2 bit 1 for 9 times then port 2 bit 2 for 8 times.

## Hints:

You can use three loops one after the other. Alter the Loop count in each one.

Use the Advanced options in the Output icon to control which output bit is used.

The examples are available directly from within the Flowcode package. From within Flowcode select: FILE…NEW PROJECT…OPEN EXAMPLE OR TEMPLATE…and select the example you want from the relevant worksheet pack.

</worksheet>

|  | <worksheet> <filename> “CP4807-2.doc” </filename> Worksheet 2 Performing calculations |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Microcontroller brains are not wired like human being’s. This has consequences for mathematics, variables and input output processes that you need to understand. In this worksheet you explore calculations and binary and hexadecimal numbering systems. |  |  |
|  | ## Part 1 |  |  |

https://youtu.be/iEkocOsMj6Q

## Over to you:

Watch the video “Binary, Hex and Magic numbers” on the Flowcode YouTube site.

Load the file “Binary Hex and magic numbers". This is a simulation only program.

There are some commands in here that you will not yet understand. Don’t worry about that for now: just look at the results of the simulation.

## Challenges:

Simulate the program and pause it or slow it down to help you understand.

Make sure you understand conversion between Binary, Hex and Decimal.

Test yourself by selecting 3 decimal numbers between 0 and 100 and converting them to binary

Repeat the exercise but convert 3 binary 8 bit numbers into decimal.

If you are using an ESP32 microcontroller this worksheet will present difficulties as the ESP32 is a 32 bit system and chip input output connections are complex.

## Part 2

https://youtu.be/T7VX-QNLzeQ

## Over to you:

Watch the video “Flowcode Embedded: a counter” on the Flowcode YouTube site.

Load the file “Counter". Set up the hardware appropriately, download it, and make sure you understand it.

The program counts in binary on a port of the microcontroller

## Challenges:

Slow down the program by altering a delay. Make sure you understand the counting sequence.

In the Project Manager click on the variable Count to see the Variable dialogue control. Count is type Byte. Investigate the Types Bool, Int and Uint.

Alter the program so that initially Count = 1. Alter the calculation in the Loop so that Count = Count * 2 rather than Count = Count +1. Can you explain what happens when you run the program?

Alter the program so that after the 7th LED is lit up the program then lights bit 6, then bit 5 etc.

## Hints:

- The Flowcode Wiki contains information on variable types

Use the Loop count so that the first loop executes 7 times. At this point bit 7 is lit up. Add a second loop that executes 7 times but use the divide ‘/’ operator in the calculation command rather than the times ‘*’ operator.

</worksheet>

|  | <worksheet> <filename> “CP4807-3.doc” </filename> Worksheet 3 Using connection points |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | There are always multiple ways of making a program. Sometimes this is down to personal preference, sometimes this is down to good programming technique. In this exercise you explore two ways of making a program: using connection points and loops. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the results of the previous worksheet. You should have a program that lights up the LEDs on a port in sequence going from bit 0 to 7 and then bit 7 to 0.

Use connection points to make the program run forever.

## Challenges:

Using connection points start from scratch and create a traffic light program that forever loops between ‘Red’ for 10 seconds, ‘Amber’ for one second, ‘Green’ for 10 seconds, ‘Amber’ for 1 second. ‘Red’ for 10 seconds and so on.

Repeat the exercise without using any connection points – just use Loops.

Which is the easiest program to understand?

## Hints:

- Add a connection point at the start of the program.

At the end of the program add a Goto Connection point.

Traffic lights: use a single loop with Output commands and delays to achieve the result.

</worksheet>

|  | <worksheet> <filename> “CP4807-4.doc” </filename> Worksheet 4 Digital inputs |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | We need a way of controlling electronic devices locally. Often this is done in a digital context – with different types of switches. In this case we need to get the input ‘commands’ and then process this to alter the state of the microcontroller or alter its processes. In this exercise you look at how to get command data into a microcontroller. |  |  |

https://youtu.be/Xvs7-iTPXcQ

## Over to you:

Watch the video “Flowcode Embedded: digital inputs” on the Flowcode YouTube site.

Load the file “Digital inputs". Set up the hardware appropriately, download it, and make sure you understand it.

The program reflects the input on one 8 bit port to another by storing the port status into a variable and then sending that to the output of another port.

## Challenges:

Alter the program so that it works on only the least significant input bit using a Boolean variable.

Alter the program so that the state of the least significant bit input is reflected to the most significant bit of the same port.

## Hints:

- Setup a new variable called ‘input_bit0’ of Type Boolean.

- Use the Advanced options of the Input command to read the state of the switch on bit 0 into ‘input_bit0’.

- Change the Output icon using Advanced options so that it outputs ‘input_bit0’ to bit 7 of the same port as the input: so now you have an output LED on bit7 and an input switch on bit 0.

## Challenges:

Alter the program again so that the state of the second bit input switch is placed in a Boolean variable ‘input_bit1’

Alter the program so that bit 7 of the same port only lights up when both ‘input_bit0’ and ‘input_bit1’ are logic high – or 1.

## Hints:

- Use the AND operator in the Output statement: input_bit0 AND input_bit1

## Challenges:

Look up Mathematical operators in the Flowcode Wiki: select HELP…WIKI

Experiment with operators OR, XOR, and NOT and make sure you understand how they are used.

</worksheet>

|  | <worksheet> <filename> “CP4807-5.doc” </filename> Worksheet 5 Making decisions |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | The behaviour of a program and its processes will often depend on the choices made by the user via a control panel or input from sensors. The way we make decisions in a program is through the IF statement and the SWITCH statement. In this worksheet you explore how decisions are made in microcontroller programs. |  |  |

https://youtu.be/jBTPa4wSFsI

## Over to you:

Watch the video “Flowcode Embedded: making decisions” on the Flowcode YouTube site.

Load the file “Making decisions". Set up the hardware appropriately, download it, and make sure you understand it.

## Challenges:

- Alter the program so that when switch A.0 and switch A.4 are pressed all the LEDs on port B are lit.

Use a SWITCH command icon with 8 branches so that when the switch on A.0 is pressed the LED on B.7 is lit, when switch A.1 is pressed the LED on B.6 is lit etc.

## Hints:

- Use your knowledge of binary to work out the input when A0 AND A4 are pressed.

- Write down the binary input for each switch press

- Set up a SWITCH statement with 8 branches which tests for each switch press

</worksheet>

|  | <worksheet> <filename> “CP4807-6.doc” </filename> Worksheet 6 Macros / subroutines |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | In designing microcontroller systems we use code that is very repetitive and we can simplify the development by using software macros or subroutines. Pre-written libraries of code for hardware modules also save us a lot of time, and help us create more functional systems. In this worksheet you explore the use of hardware and software macros. |  |  |

https://youtu.be/hNBSxdb72v4

## Over to you:

Watch the video “Macros” on the Flowcode YouTube site.

Load the file “how to use macros". Set up the hardware appropriately, download it, and make sure you understand it.

## Challenges:

Alter the program so that it Prints “Count is: “ then the count number

Alter the program so that it prints the number on the second line of the display rather than the first

Assign the text “Count is: “ to a variable and use the string variable in your program.

Add a second software macro to the program called “Initialise”.

Move the Start command to this macro and then call it from the Main routine.

## Hints:

- Use the hardware macro PrintString to print a string (you need to use inverted commas in the dialogue box)

- Add a new variable “mystring” of Type String and length 20. Use PrintString to print this.

- Use the Cursor hardware macro to alter the print position.

## Challenges:

Develop a program that uses the 7-segment displays on the Combo board to display the value of Count.

Create a document that tells someone new to your project how it works.

## Hints:

- Use a BL0114 Combo board component from the Hardware group and the Seg_Showdigit macro

- Use three variables: Units, Tens Hundreds.

- Use some maths to convert Count into Units, Tens and Hundreds - Investigate the MOD mathematical function – see the Wiki for details.

- Use the FILE….EXPORT…as HTML option to create a HTML document of your project. There is a lot of information here. Identify the parts you want and use copy and paste to assemble a report on what you have done.

</worksheet>

|  | <worksheet> <filename> “CP4807-7.doc” </filename> Worksheet 7 Using prototype boards |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | So far you have developed programs using boards which are manufactured for you. At some point you will need to create your own circuits. You can do this with a prototype board. This links to E-Blocks very easily to provide you with the connections and power supplies that you need. In this worksheet you explore the use of prototype boards in microcontroller circuits. |  |  |

There is no video for this worksheet.

## Over to you:

The diagrams above show you a circuit that you need to build. Two switches (active high and active low), a single LED and a buzzer.

Build this circuit and add it to your upstream E-blocks system on any port.

Develop a program that:

  - When both switches are pressed activates the buzzer.

  - Flashes the LED once a second

## Hints:

- You can start from scratch of start from a previous program – perhaps the one on Digital inputs

- The main problem here is that there are two functions which need to be carried out at the same time. It is difficult to do both at once. (later you will learn how to use interrupts for this situation). Set up a loop with a 10ms delay. 100 of these loops gives you 1 second. Set up a variable to track a second has elapsed. Use this to change the state of the LED output.

- Use the && operator for the logic of both switches being pressed at once. One of your switches is active high and the other active low.

</worksheet>

|  | <worksheet> <filename> “CP4807-8.doc” </filename> Worksheet 8 Colour graphical displays |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Graphical displays extend the functionality of electronic systems, allow more information about be displayed, and allow information to be displayed in more easily understood formats: tables, graphics, graphs etc. In this worksheet you explore how colour graphical displays work. |  |  |

https://youtu.be/wBFTZGid_Ck

## Over to you:

Watch the video “Colour graphical displays” on the Flowcode YouTube site.

Load the program “Colour graphical displays”. Set up the hardware appropriately, download it, and make sure you understand how the program works.

## Challenges:

Create a program that uses the colour graphical display to show the status of 8 of the switches on the Combo board and the value of a potentiometer on an XY graph.

Your display will look something like this:

## Hints:

- Use 8 ellipses to show the state of the switches. Each ellipse has a ModifyColour macro you can use to change ellipse colour – use red (255,0,0) for switch off and green (0, 255, 0) for switch on.

Use the graph component to show the value of the potentiometer. You may need to complete the first worksheet in Sensors and Microcontrollers to understand how to do get a value from a potentiometer.

Use text fields appropriately to label your components.

Modify colours on all components to make your screen look great.

If you can; create a bitmap using a graphics package and use the Bitmap drawer component to display it.

Refresh the graph every 60 seconds – you don’t need to clear the whole display.

</worksheet>

|  | <worksheet> <filename> “CP4807-9.doc” </filename> Worksheet 9 Pin interrupts |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | In some systems there are possible events that are really important – like the kill switch on a factory machine. When switches like this are activated you need your program to stop whatever it is doing – ‘interrupt’ the program - and take action. Interrupts allow us to do this. In this worksheet you explore how pin interrupts work. |  |  |

https://youtu.be/imTq19jX408

## Over to you:

Watch the video “Flowcode Embedded: pin interrupts” on the Flowcode YouTube site.

Load the file “pin interrupts". Set up the hardware appropriately, download it, and make sure you understand it.

## Challenges:

Alter the program so that a LED output flashes during normal operation.

Alter the program so that once the ‘stop switch’ on the interrupt pin is activated the program waits until a ‘clear’ switch is pushed before continuing the program.

## Hints:

- Put a flashing LED routine in the main loop.

In the Interrupt routine insert a While…until loop.

Use a variable to track the status of the ‘clear’ switch.

In the loop test for the ‘clear’ switch being pressed.

</worksheet>

|  | <worksheet> <filename> “CP4807-10.doc” </filename> Worksheet 10 Timer interrupts |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Many microcontroller systems need some kind of timing system. Microcontrollers are designed to help us here as they have dedicated internal hardware that counts the number of internal clock pulses and converts them into time. When activated these internal counters can ‘interrupt’ the program to help you manage time-based code. In this worksheet you explore how timer interrupts work. |  |  |

https://youtu.be/E5SWKxIjvno

## Over to you:

Watch the video “Flowcode Embedded: timer interrupts” on the Flowcode YouTube site.

Load the file “timer interrupts". Set up the hardware appropriately.

## Challenges:

Alter the program so that it calculates the hours, minutes and seconds that have elapsed.

Use a switch and a pin interrupt to clear the timer.

## Hints:

- Create variables hour_count and minute_count

In the main loop use IF statements to track when second_count is more then 60 and reset it to 0 and increment minute_count Similarly for hour_count.

Keep the programming in the interrupt routine to a minimum – it is good practice

Create a new routine for a pin interrupt and use the code from the previous exercise to create the reset function.

</worksheet>

|  | <worksheet> <filename> “CP4807-11.doc” </filename> Worksheet 11 Touch control systems |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Touchscreen systems are replacing the use of switches, potentiometers, LEDs and other electronic devices. Using a graphical touch screen reduces costs and enhances functionality. In this worksheet you learn how to develop a system that uses the touch screen display to control a simple system. |  |  |

https://youtu.be/vS7P8RzybKc

## Over to you:

Watch the video “Touch screen control” on the Flowcode YouTube site.

Load the file “Touch screen control”. Set up the hardware appropriately.

Download the file and make sure you understand how the program works.

## Challenges:

Take the program you wrote in the Colour Graphical Displays program.

Modify it so that it has two touch buttons: ‘+’ and ‘-‘ and a text field

Use these to control an output variable and show this variable (between 0 and 255) on a port of 8 LEDs.

Your display will look something like this:

Hints:

- Use a new variable: Output

- Use the touch screen routine to detect when the ‘+’ and ‘-‘ text fields are touched and modify the variable Output accordingly

- Write the Output variable to a port to show its value.

- Note that the DC_touched macro is called by each display primitive as it is touched – this is set in the Touch callback property

</worksheet>

|  | <worksheet> <filename> “CP4807-12.doc” </filename> Worksheet 12 Web mirror |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Touchscreen systems are replacing the use of switches, potentiometers, LEDs and other electronic devices. Using a graphical touch screen reduces costs and enhances functionality. In this worksheet you learn how to develop a more complex touch screen system |  |  |

https://youtu.be/rdSOYW6AXn0

## Over to you:

Watch the video “Web mirror” on the Flowcode YouTube site.

Load the file “Web mirror”. Set up the hardware appropriately. Change the EB3 Web Mirror settings of the Touch screen component and enter the name and password of your local server.

Download the file and make sure you understand how the program works.

Use your mobile phone camera to grab the URL of the remote server from the displayed QR code that allows you to control the system over the internet.

## Challenges:

Take the program you wrote in the Touch control systems worksheet.

Change the EB3 Web Mirror settings of the Touch screen component to ‘Yes’ and enter the name and password of your local server.

Compile the program to hardware.

Make sure that you can use your program on your mobile phone to control and monitor your hardware.

</worksheet>

|  | <HTML> <filename> “CP4807-TN.htm” </filename> Teacher’s notes | Introduction to microcontrollers |
| --- | --- | --- |

## Previous learning

Some understanding of electronics is needed.

## Pedagogy

This set of worksheets provides structure for the study of microcontroller programming. The worksheets provide structure to the learning but do not provide all the information students need. Our vision as to how you teach this subject is:

- This is a very practical subject. We expect that 90% of the time students spend on this module will be spent working alone.

- Each student should have Flowcode Embedded software and a set of E-blocks 3 hardware. Don’t be tempted to try to get students to learn this subject in pairs.

- Students should be introduced to each topic or worksheet. They should be given time to investigate the topic using the internet. If students are investigating this on their own, you may want to give additional structure to this activity.

- Students should then load an example program and download this to the microcontroller so that they can see how the program works. Example programs are available on the Flowcode Wiki for each of PIC, Arduino Mega and ESP32 microcontrollers.

- Students should be given the opportunity to watch the accompanying YouTube video – individually or as a class activity. This explains the program.

- Students then modify each example as detailed in the worksheet. This ensures that they have success, but also ensures that they have understanding. Our scheme of work includes 3 assessments and a final project.

- Students do not have to do all worksheets or tasks. That’s up to you to decide. Worksheet 10 and 11 are quite advanced.

- There are three sets of worksheets in this series: Introduction to microcontrollers, Sensors and microcontrollers, and Motors and microcontrollers. We have split the content by topic for ease of publication. You will need to decide what sequence your students complete the worksheets in. We have split the worksheet difficulty into Bronze, Silver, and Gold. Not all students will be able to complete all exercises. A reasonable approach would be to deliver Bronze in each of Introduction to microcontrollers, Sensors and microcontrollers, and Motors and microcontrollers, then deliver Silver then deliver Gold to those students who are capable.

- The curriculum is based on the requirements of the BTEC level 3 in Engineering – Unit 6: Microcontroller systems for engineers.  This can be downloaded from the Pearson web site.

- We have included – below – a full scheme of work for a 60 hour module in microcontroller development.

## Example programs

In general the worksheets make use of prewritten functional examples. Students load the example, understand it and then modify it to make sure that they have learned  the principles under discussion.

The examples are available directly from within the Flowcode package. From within Flowcode select: FILE…NEW PROJECT…OPEN EXAMPLE OR TEMPLATE…and select the example you want from the relevant worksheet pack.

Examples are available separately for PIC, Arduino and ESP32 microcontrollers.

## Time

Of course this depends on student capability. We estimate that it will take 15 hours to complete all the worksheets in this module.

## Software

The videos make use of Flowcode embedded and we anticipate that you are likely to use this for your programs. But you can use these worksheets with any microcontroller development software. The flowchart programs can easily be followed by C programmers.

## Assessment

Creating programs from scratch will be suitable as a way of assessing that students have understood the programs and have programming ability.

## Hardware

This course is designed to be used with a Matrix Microcontroller development centre. There are three versions:

BL5394		ESP32 microcontroller development centre

BL8624		PIC microcontroller development centre

BL3797		Arduino microcontroller development centre

These consist of:

Upstream board – BL0082 PIC, BL0040 Arduino, or BL0070 ESP32

BL0114 Combo board

BL0117 Prototype board

BL0118 Project board

BL0127 Actuators board

BL0135 9 axis motion / accelerometer board

BL0144 Temp/humidity board

BL0145 Switch board

BL0156 Splitter board

BL0172 Logic analyser with ribbon cable

BL0167 LED board

BL0183 Relay board

BL0189 Analogue board

List of all tutorials across workbooks

Introduction to microcontrollers

Bronze

Worksheet 1 		First program

Worksheet 2		Performing calculations

Worksheet 3		Connection points

Worksheet 4		Digital inputs

Worksheet 5		Making decisions

Worksheet 6		Macros / subroutines

Worksheet 7		Using prototype boards

Silver

Worksheet 8		Colour graphical displays

Worksheet 9		Pin interrupts

Worksheet 10		Timer interrupts

Gold

Worksheet 11		Touch control systems

Worksheet 12		Web mirror

Sensors and Microcontrollers

Bronze

Worksheet 1 		Analogue inputs

Worksheet 2		Light sensor

Worksheet 3 		Analogue temperature sensor

Worksheet 4		Digital temperature sensor

Worksheet 5		Digital accelerometer

Silver

Worksheet 6		Floats and ints

Gold			Any one of the following mechanical sensor projects:

Additional worksheets for mechanical sensors. These can be used as worksheets or projects.

Worksheet 7		Thermocouple

Worksheet 8		Flow sensor

Worksheet 9		Compressive force sensor

Worksheet 10		Strain sensor

Worksheet 11		Pressure sensor

Motors and microcontrollers

Bronze

Worksheet 1 		Basic DC motor control

Worksheet 2		Full bridge motor control

Worksheet 3		Servo motor control

Worksheet 4		Stepper motor control

Gold

Worksheet 5		DC motor speed control

PC interfacing

Bronze

Worksheet 1 		Beginning hardware interfacing – PC to hardware

Worksheet 2		Bidirectional hardware control

Worksheet 3 		JSON encoding

Silver

Worksheet 4		Full PC – Embedded project

</HTML>

Homework assignments

<HTML> <filename> “CP4807-H1.htm” </filename>

Homework 1

First program

Performing calculations

Digital inputs

Using connection points

Make sure that you can replicate the examples from scratch. This make sure that you understand them.

Design brief:

Design a program that controls two sets of traffic lights at a junction.

Hardware:

Upstream board

Use a Combo board on port A and Port B where the connections to the traffic lights are represented by:

Port A bit 0 GREEN

Port A bit 1 AMBER

Port A bit 2 RED

Port B bit 0 GREEN

Port B bit 1 AMBER

Port B bit 2 RED

Software

Create a panel in Flowcode with the two sets of lights in the appropriate colour. You can do this using the single LED component. You can carry out this program in simulation and then try it on the hardware next time you are in class.

Design the program so that the lights at the junction control the traffic in an appropriate sequence of:

RED: 20 seconds

AMBER: 3 seconds

GREEN: 20 seconds

When there is RED at one set of lights there must be GREEN at the other.

</HTML>

<HTML> <filename> “CP4807-H2.htm” </filename>

Homework 2

Making decisions

Macros and subroutines

Colour graphical displays

Design brief:

Carry on with the worksheet on Colour Graphical displays.

Hardware:

Upstream board

Analogue Sensor board with potentiometer on the appropriate port (Port F for Arduino and any port for PIC and ESP)

Switch board on port A

Software

You should have created a panel with 8 switches and a potentiometer. You can carry out this program in simulation and then try it on the hardware next time you are in class.

</HTML>

Homework 3

<HTML> <filename> “CP4807-H3.htm” </filename>

Colour graphical displays

Analogue inputs

Light sensor

Design brief:

Design a program that controls two sets of traffic lights at a junction.

Hardware:

Upstream board

Analogue Sensor board with light sensor on the appropriate port (Port F for Arduino and any port for PIC and ESP)

Software

Create a program that reads in the value of the light sensor.

Dedicate an area of the Colour display to be a ‘light’.

Draw a colour rectangle in the dedicated area which is inversely proportional to the value from the light sensor. The colour of the rectangle is dictated by R, G, B values and you can use these to dictate the colour.

Add text to your display to give your project a name.

</HTML>

Homework 4

<HTML> <filename> “CP4807-H4.htm” </filename>

Digital accelerometer

Design brief:

Carry on with the worksheet on the digital level.

Hardware:

Upstream board

Accelerometer sensor

Software

Show the level with a display on the bottom of the screen – use a moving circle to show the level. The circle will be in the centre of the display when level.

On the right of the screen show the vertical component –the circle will be in the centre when Z = 0.

</HTML>

Homework 5

<HTML> <filename> “CP4807-H5.htm” </filename>

Work on Assessment 2

</HTML>

Homework 6

<HTML> <filename> “CP4807-H6.htm” </filename>

DC motor control

Full bridge control

Design brief:

Create a program that uses a potentiometer to control the speed of the motor.

Hardware:

Upstream board

Combo board or switch board

Actuators board

Software

Use two switches to control whether the motor turns forwards or backwards. The switches are push to make, so you will need to use logic to detect a switch is pressed and then set a DIRECTION flag. Use the Colour graphical display to show the user what is happening in the program.

</HTML>

Homework 7

<HTML> <filename> “CP4807-H7.htm” </filename>

Timer interrupts

Design brief:

Create a stopwatch program.

Hardware:

Upstream board

Combo board or switch board

Software

Create a stopwatch program with the following features:

A single start/stop switch

A reset switch

Colour graphical display with the time in 7-segment font in the centre of the display

Display nicely formatted with title and well designed.

</HTML>

Homework 8

<HTML> <filename> “CP4807-H8.htm” </filename>

Touch control systems

Design brief:

Carry on with the worksheet on the touch screen project.

Hardware:

Upstream board

Combo board

Software

As dictated in the Challenge section in the worksheet.

</HTML>

Homework 9, 10

<HTML> <filename> “CP4807-H9.htm” </filename>

Students need to pan their project.

The plan should have the following sections:

Design brief:

A full design brief with sketches of the panel, the hardware, and a sketch of the Colour touch screen display graphics if this is used. Planning is important.

Hardware:

A description of the hardware used. E-blocks boards used and any prototype board circuitry.

Software

A description of the software functionality. Partly described by the Colour touch screen sketches.

Technical risk

A list of the technical risks in the project – what does the students not know? What are the areas of difficulty? How will the student overcome these difficulties? What does the student need to learn.

</HTML>

Assessments

<HTML> <filename> “CP4807-A1.htm” </filename>

Assessment 1

This first assessment is designed to test students on what they have learned so far. The project is one that they have carried out before – a simple light sensor and a section on the colour display that acts as a ‘light’.

Students should start the program from scratch rather than loading previous work. This will require some supervision. IF students have done this work then this will reinforce their understanding and it will giver supervisors a good idea of which students are on top of the subject and which are not.

Design brief:

Design a program that controls two sets of traffic lights at a junction.

Hardware:

Upstream board

Analogue Sensor board with light sensor on the appropriate port (Port F for Arduino and any port for PIC and ESP)

Software

Create a program that reads in the value of the light sensor.

Dedicate an area of the Colour display to be a ‘light’.

Draw a colour rectangle in the dedicated area which is inversely proportional to the value from the light sensor. The colour of the rectangle is dictated by R, G, B values and you can use these to dictate the colour.

Add text to your display to give your project a name.

</HTML>

Assessment 2

<HTML> <filename> “CP4807-A2.htm” </filename>

This is part in classroom and part homework. A 6 hour block  of time that acts as a mini project.

Design brief:

Create a digital temperature controller that allows you to control the temperature in a room.

Hardware:

Upstream board

Combo board or switch board

Digital temperature sensor board

Relay board

Software

Design the Colour graphical display with large characters for the current temperature as measured by the sensor board.

Below the main display have a second text field with the target temperature.

Use two switches, up and down, to control the target temperature value and display. Update the target temperature as the switches are pressed.

When the actual temperature is greater than the target temperature the heater is OFF. When the actual temperature is less than the target temperature the heater is ON.

Use a text field on the top right of the display to show the heater status: ON or OFF.

Use a single LED to display the heater status.

Activate the relay when the heater should be on.

</HTML>

<HTML> <filename> “CP4807-A3.htm” </filename>

Assessment 3

This is part in classroom and part homework. A 6 hour block of time that acts as a mini project.

Design brief:

Enhance the digital temperature controller with touch functionality.

Hardware:

Upstream board

Combo board or switch board

Digital temperature sensor board

Relay board

Software

Design the Colour graphical display with large characters for the current temperature as measured by the sensor board.

Below the main display have a second text field with the target temperature.

Use two virtual switches on the touch screen for UP and DOWN.

When the actual temperature is greater than the target temperature the heater is OFF. When the actual temperature is less than the target temperature the heater is ON.

Use a text field on the top right of the display to show the heater status: ON or OFF.

Use a single LED to display the heater status.

Activate the relay when the heater should be on.

Add a third touchable area on the display that acts as an Over ride button to turn the heater on for 10 minutes even when the actual temperature is above the set temperature.

Add a field to the display that shows the time using a timer interrupt.

</HTML>

<HTML> <filename> “CP4807-P1.htm” </filename>

Project

Students should develop a project of their own which proves their capability. They should discuss this with the teacher.

Design brief:

A full design brief with sketches of the panel, the hardware, and a sketch of the Colour touch screen display graphics if this is used. Planning is important.

Hardware:

A description of the hardware used. E-blocks boards used and any prototype board circuitry.

Software

A description of the software functionality. Partly described by the Colour touch screen sketches.

Technical risk

A list of the technical risks in the project – what does the students not know? What are the areas of difficulty? How will the student overcome these difficulties? What does the student need to learn.

</HTML>

<HTML> <filename> “CP4807-marking scheme.htm” </filename>

Assessment marking schemes

Bronze (Functional & Basic): The project works but may be brittle, poorly documented, or lack error handling. It meets minimum requirements.

Silver (Robust & Structured): The code is clean, well-structured, follows standards, and includes documentation/tests. It is "developer-ready".

Gold (Polished & Production-Ready): The project is optimized, highly scalable, and user-ready. It includes comprehensive testing, documentation, and handles edge cases gracefully. It exceeds the design brief and shows real design flair.

</HTML>

<Document> <filename> “CP4807-CPDcert” </filename>

# This is to certify that:

# <name>

# Has completed a Continuous Professional Development training course on E-blocks 3 and Flowcode

# On:

# <Date>

# Signed:

# The Matrix team.

</Document>

Scheme of work – 60 hour

<HTML> <filename> “CP4807-SOW.htm” </filename>

| Session | Objectives/learning outcomes The learner will: | Activities and resources | Assessment criteria |
| --- | --- | --- | --- |
| 1 2 hours | Introducing the course, content, scheme of learning, hardware, software, assessment, grading, getting each student up and running with equipment – making sure drivers are installed and hardware and software are working | Display PowerPoint presentation Get equipment out - Resources: PowerPoint presentation Microcontroller programming, Microcontroller basics document / Powerpoint - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 2 2 hours | First program Performing calculations | Look at the workshsheet. Look at the videos – play videos in front of class or reproduce content ‘live’. Students carry out challenges in the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 3 2 hours | Digital inputs Using connection points | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| Homework 1 2 hours | First program Performing calculations Digital inputs Using connection points | Homework 1 assignment - Resources: Introduction to microcontroller programming worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Homework Programs submitted by email |
| 4 2 hours | Making decisions Macros and subroutines . | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | None |
| 5 2 hours | Prototype boards | Show the starting videos to the whole class. Discuss prototype board function more. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki - Resources: Resistors, LEDs, switches etc for prototype boards | None |
| 6 2 hours | Analogue inputs | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Sensors and microcontrollers worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 7 2 hours | Colour graphical displays | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| Homework 2 2 hours | Making decisions Macros and subroutines Colour graphical displays | Homework 2 assignment - Resources: Introduction to microcontroller programming worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Homework Programs submitted by email |
| 8 2 hours | Colour graphical displays | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 9 2 hours | Light sensor | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Sensors and microcontrollers worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| Homework 3 2 hours | Colour graphical displays Analogue inputs Light sensor | Homework 3 assignment - Resources: Sensors and microcontrollers worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Homework Programs submitted by email |
| 10 2 hours | Assessment 1 | - Resources: Assessment 1 - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 11 2 hours | Digital temperature sensor | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Sensors and microcontrollers worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 12 2 hours | Digital accelerometer | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Sensors and microcontrollers worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| Homework 4 2 hours | Digital accelerometer | Homework 4 assignment - Resources: Sensors and microcontrollers worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Homework Programs submitted by email |
| 13 2 hours | Assessment 2 | - Resources: Assessment 2 - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| Homework 5 2 hours | Assessment 2 | - Resources: Assessment 2 - Resources: Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 14 2 hours | Assessment 2 | - Resources: Assessment 2 - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 15 2 hours | Basic DC motor control Full bridge control | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Motors and microcontrollers worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | None |
| 16 2 hours | Stepper motor control Servo motor control | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Motors and microcontrollers worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | None |
| Homework 6 2 hours | DC motor control | - Resources: Homework 6 assignment - Resources: Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Homework Programs submitted by email |
| 17 2 hours | Floats and Ints | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Sensors and microcontrollers worksheets - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | None |
| 18 2 hours | Pin interrupts | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 19 2 hours | Timer interrupts | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| Homework 7 4 hours | Timer interrupts | - Resources: Homework 7 assignment - Resources:. E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Homework Programs submitted by email |
| 20 2 hours | Touch control systems | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 21 2 hours | Touch control systems | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 22 2 hours | Assessment 3 | - Resources: Assessment 3 - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| Homework 8 4 hours | Touch control systems | - Resources: Homework 8 assignment - Resources:.Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | Homework Programs submitted by email |
| 23 2 hours | Web mirror | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| 24 2 hours | Web mirror | Show the starting videos to the whole class. Q &A. Students carry out the worksheet. - Resources: Introduction to microcontroller programming worksheets - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | None |
| Homework 9 2 hours | Project | Students plan their project - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | none |
| 25 2 hours | Project | Student lead project. - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 26 2 hours | Project | Student lead project. - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 27 2 hours | Project | Student lead project. - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 28 2 hours | Project | Student lead project. - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 29 2 hours | Project | Student lead project. - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| 30 2 hours | Project | Student lead project. - Resources: E-blocks hardware. Flowcode software - Resources: Flowcode YouTube site | Gold, Silver, Bronze detailed in Assessment. Programs submitted by email |
| Homework 9 10 hours | Project | Students work on their project at home - Resources:. Flowcode software - Resources: Flowcode YouTube site - Resources: Flowcode Wiki | none |

</HTML>

Scheme of work – 120 hour

The 120 hour scheme of work includes the 60 hour scheme of work and the following:

Allcode competition

Formula Allcode is a small mobile robot powered by a PIC microcontroller. Allcode is a fantastic tool for teaching Autonomous mobile robotics through a range of exercises including line following, tune playing and dancing and maze following.

PC interfacing

In this module students learn how to create apps for a PC which interface to low cost embedded systems like Arduino Uno.

Working with mechanical sensors

Working with strain sensors, Thermistors, pressure sensors, and power boards.

DC motor control

Students understand how to control DC motors using feedback.

Further project

Students undertake a larger project.

## BTEC National Unit 6 – topic coverage

# Summary of outputs from this document

Lots of files created from this document

CP4807- Introduction to microcontrollers worksheets.pdf

CP4807 – Introduction to microcontrollers browser based course

CP4807 – Introduction to microcontrollers SCORM content course

CP4807 – Introduction to microcontrollers teacher CPD

# AI splitter instructions

<AI splitter instructions>

<command> “Go through this document and create individual files with the content that starts with a tag enclosed in back and forward ticks <> as follows:

Content between “[HTML]” and “[/HTML]”: make me an HTML file from the content between these tags

Content between “[worksheet]” and “[/worksheet]”: make me a Word file from the content between these tags

Content between “[document]” and “[/document]”: make me a Word file from the content between these tags

</command>

<AI splitter instructions>

# AI instructions for Introduction to microcontrollers worksheets.pdf

<AI instruction>

“CP4807- worksheets.pdf”

<Command>

“Please make me a single PDF document consisting of the following documents:

“CP4807-head.docx”

“CP4807-cont.docx

“CP4807-1.docx”

“CP4807-2.docx”

“CP4807-3.docx”

“CP4807-4.docx”

“CP4807-5.docx”

“CP4807-6.docx”

“CP4807-7.docx”

“CP4807-8.docx”

“CP4807-9.docx”

“CP4807-10.docx”

“CP4807-11.docx”

“CP4807-12.docx”

“CP4807-TN.docx”

“CP4807-SOW.htm”

”

</command>

</AI instruction>

Questions: How to take fonts and stuff from a single definition file. Do we need to?

# AI instructions for Introduction to microcontrollers browser based course

<AI instruction>

<Command>

“Please make me a browser-based course with the following screens:

Screen type	Title				File

Opening screen	Introduction to microcontrollers	“CP4807 – opening.png”

HTML		Welcome			“CP4807 – welcome.HTM”

YouTube		Introducing E-blocks 3		https://youtu.be/KmpyVmv6J_Y?si=bzAVMntlAOE_-Rlv

YouTube		Introdcing Flowcode		https://youtu.be/tDdptTbvDM0?si=19xqKAzPCOO3In5e

PDF		E-blocks datasheet		https://www.matrixtsl.com/wp-content/uploads/2026/03/CP9645-Eblocks-3-Datasheet-1.pdf

Powerpoint	Microcontroller basics 1		“Microcontroller basics 1 24 02 26.pptx”

Powerpoint	Microcontroller basics 2		“Microcontroller basics 2 24 02 26.pptx”

Document	First program 			“CP4807-1.docx”

Document 	Performing calculations 		“CP4807-2.docx”

Document 	Connection points 		“CP4807-3.docx”

Document 	Digital inputs 			“CP4807-4.docx”

HTML 		Homework 1			“CP4807-H1.htm”

Document 	Making decisions			“CP4807-5.docx”

Document 	Macros / subroutines 		“CP4807-6.docx”

Document 	Using prototype boards		“CP4807-7.docx”

Document	Analogue inputs			“CP1972-1.docx”

Document 	Colour graphical displays		“CP4807-8.docx”

HTML 		Homework 2			“CP4807-H2.htm”

Document 	Colour graphical displays		“CP4807-8.docx”

Document	Light sensor			“CP1972-2.docx”

Document	Analogue temperature sensor	“CP1972-3.docx”

HTML 		Homework 3			“CP4807-H3.htm”

HTML 		Assessment 1			“CP4807-A1.htm”

Document	Digital temperature sensor		“CP1972-4.docx”

Document	Digital accelerometer		“CP1972-5.docx”

HTML 		Assessment 2			“CP4807-A2.htm”

Document	Basic DC motors			“CP0507-1.docx”

Document	Full bridge motor control		“CP0507-2.docx”

Document	Stepper motor control		“CP0507-3.docx”

Document	Servo motor control		“CP0507-4.docx”

HTML 		Homework 6			“CP4807-H6.htm”

HTML 		Floats and INTs			“CP1972-6.docx”

Document 	Pin interrupts			“CP4807-9.docx”

Document 	Timer interrupts			“CP4807-10.docx”

HTML 		Homework 7			“CP4807-H7.htm”

Document 	Touch control systems		“CP4807-11.docx”

HTML 		Homework 8			“CP4807-H8.htm”

Document 	Web mirror			“CP4807-12.docx”

HTML 		Homework 9			“CP4807-H9.htm”

“

</command>

</AI instruction>

Questions: Somehow want to define a single engine: we don’t want lots of AI programs. We want consistency. Screen definitions we perhaps also need to define the types of screen we have?

# AI instructions for Introduction to microcontrollers SCORM content course

I need some help here.

<manifest>

identifier

version

xml:base

<metadata>

<schema>

<schemaversion>

<lom or adlcp:location

<organisations>

default

<organisation>

identifier

structure

adlseq:objectivesGlobalToSystem

<title>

<item>

identifier

identifierref

isvisible

parameters

<title>

<item>

<metadata>

<lom or adlcp:location>

<adlcp:timeLimitAction>

<adlcp:dataFromLMS>

<adlcp:completionThreshold>

completedByMeasure

minProgressMeasure

progressWeight

<imsss:sequencing>

<adlnav:presentation>

<navigationInterface>

<hideLMSUI>

<metadata>

lom or adlcp:location

<imsss:sequencing>

<resources>

identifier

type

href

xml:base

adlcp:scormType

<metadata>

<lom or adlcp:location>

<file>

href

<metadata>

<lom or adlcp:location>

<dependency>

identifierref

<imsss:sequencingCollection>

<sequencing>

<AI instruction>

<Command>

“Please make me a SCORM file with the following screens:

Screen type	Title				File

Opening screen	Introduction to microcontrollers	“CP4807 – opening.png”

HTML		Welcome			“CP4807 – welcome.HTM”

YouTube		Introducing E-blocks 3		https://youtu.be/KmpyVmv6J_Y?si=bzAVMntlAOE_-Rlv

YouTube		Introdcing Flowcode		https://youtu.be/tDdptTbvDM0?si=19xqKAzPCOO3In5e

PDF		E-blocks datasheet		https://www.matrixtsl.com/wp-content/uploads/2026/03/CP9645-Eblocks-3-Datasheet-1.pdf

Powerpoint	Microcontroller basics 1		“Microcontroller basics 1 24 02 26.pptx”

Powerpoint	Microcontroller basics 2		“Microcontroller basics 2 24 02 26.pptx”

Document	First program 			“CP4807-1.docx”

Document 	Performing calculations 		“CP4807-2.docx”

Connection points 		“CP4807-3.docx”

Document 	Digital inputs 			“CP4807-4.docx”

HTML 		Homework 1			“CP4807-H1.htm”

Document 	Making decisions			“CP4807-5.docx”

Document 	Macros / subroutines 		“CP4807-6.docx”

Document 	Using prototype boards		“CP4807-7.docx”

Document	Analogue inputs			“CP1972-1.docx”

Document 	Colour graphical displays		“CP4807-8.docx”

HTML 		Homework 2			“CP4807-H2.htm”

Document 	Colour graphical displays		“CP4807-8.docx”

Document	Light sensor			“CP1972-2.docx”

Document	Analogue temperature sensor	“CP1972-3.docx”

HTML 		Homework 3			“CP4807-H3.htm”

HTML 		Assessment 1			“CP4807-A1.htm”

Document	Digital temperature sensor		“CP1972-4.docx”

Document	Digital accelerometer		“CP1972-5.docx”

HTML 		Assessment 2			“CP4807-A2.htm”

Document	Basic DC motors			“CP0507-1.docx”

Document	Full bridge motor control		“CP0507-2.docx”

Document	Stepper motor control		“CP0507-3.docx”

Document	Servo motor control		“CP0507-4.docx”

HTML 		Homework 6			“CP4807-H6.htm”

HTML 		Floats and INTs			“CP1972-6.docx”

Document 	Pin interrupts			“CP4807-9.docx”

Document 	Timer interrupts			“CP4807-10.docx”

HTML 		Homework 7			“CP4807-H7.htm”

Document 	Touch control systems		“CP4807-11.docx”

HTML 		Homework 8			“CP4807-H8.htm”

Document 	Web mirror			“CP4807-12.docx”

HTML 		Homework 9			“CP4807-H9.htm”

“

</command>

</AI instruction>

# AI instructions for Introduction to microcontrollers teacher CPD browser based course

<AI instruction>

<Command>

“Please make me a browser-based course with the following screens:

Screen type	Title				File

Opening screen	Introduction to microcontrollers	“CP4807 – opening.png”

HTML		Welcome			“CP4807 – CPD objectives.HTM”

YouTube		Introducing E-blocks 3		https://youtu.be/KmpyVmv6J_Y?si=bzAVMntlAOE_-Rlv

YouTube		Introdcing Flowcode		https://youtu.be/tDdptTbvDM0?si=19xqKAzPCOO3In5e

PDF		E-blocks datasheet		https://www.matrixtsl.com/wp-content/uploads/2026/03/CP9645-Eblocks-3-Datasheet-1.pdf

Powerpoint	Microcontroller basics 1		“Microcontroller basics 1 24 02 26.pptx”

Powerpoint	Microcontroller basics 2		“Microcontroller basics 2 24 02 26.pptx”

Document	First program 			“CP4807-1.docx”

Document 	Performing calculations 		“CP4807-2.docx”

Connection points 		“CP4807-3.docx”

Document 	Digital inputs 			“CP4807-4.docx”

HTML 		Homework 1			“CP4807-H1.htm”

Document 	Making decisions			“CP4807-5.docx”

When the client has gone through all screens and marked them as complete please print them a certificate of completion using “CP4807-CPDcert.docx”

And insert the candidate name in the <name> field and the date in the <date> field.

“

</command>

</AI instruction>

|  | Version control | Introduction to microcontrollers |
| --- | --- | --- |

04 12 25	First release

18 02 26	Second release with schemes of work

05 04 26 	with AI mark up instructions