|  | Worksheet 9 Compressive force sensors |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Force sensors are used with some mechanical attachment for measuring weight. In this worksheet / project you learn how compressive force sensors are used to measure weight. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0132 Compression - Forsentek". Set up the hardware appropriately.

Make sure you understand how the program works. To start with you can compress the sensors to get a measurement.

## Challenge:

- The compressive and tensile load cell, works by deflecting the internal strain gauges. These type of sensors are great in applications for axial loads. For example, reading forces applied to a structural member of a frame. Most of these sensors come with a threaded bolt hole down the vertical axis of the load cell for attachment.

Make a mechanical rig that compresses the sensor when weights are placed on the rig.

Modify the program to make a weighing scale with the following characteristics:

  - Fore sensor with a platform

  - A display showing the weight of the object

  - A zero switch or touch object

## Hints:

- Review the worksheets in Introduction to microcontrollers where you will find examples of touchable buttons for graphical displays.

<\worksheet>

|  | Worksheet 10 Cantilever strain sensors |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Cantilever strain sensors are extensively used to measure forces in mechanical systems. The choice of compressive or cantilever sensors is governed by the mechanical properties of the system being measured. In this worksheet / project you learn how Cantilever sensors are used to measure weight. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0132 Compression - Tedea". Set up the hardware appropriately.

Make sure you understand how the program works. To start with you can activate the sensor with your hands to get a measurement.

## Challenge:

The diagram shows a typical cantilever setup. You can set up the cantilever load cell provided in such a way that that it represents a weighting scale. Design yourself a base which can be clamped down or won’t move. Raise the load cell so that there is clearance for deflection. Then design a platform which will attach to the top side of the load cell, so that you can balance items over it.

Modify the program to make a weighing scale with the following characteristics:

  - Cantilever strain sensor

  - A display showing the weight of the object

  - A zero switch or touch object

## Hints:

- Review the worksheets in Introduction to microcontrollers where you will find examples of touchable buttons for graphical displays.

<\worksheet>

|  | Worksheet 11 Pressure sensors |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Pressure sensors are used extensively in the automotive industry for tyre pressure monitoring in vehicles, in carburettors, and exhaust systems. In this worksheet / project you learn how to use pressure sensors in microcontroller circuits. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0122 Pressure". Set up the hardware appropriately.

Make sure you understand how the program works. To start with you can blow, or suck, on the sensor to make sure it works.

## Challenge:

Get hold of a syringe style pump and hose to attach to the sensor.

Modify the program to make a pressure measuring system with the following characteristics:

  - Microcontroller system, sensor, pump and hose

  - A display of pressure rate, set flow rate and pump status

  - A graph showing flow rate variation over time – refreshing every 30 seconds. Temperature range 15C to 30C.

  - An LED that lights up when the target flow rate is over run

  - A way of altering the set flow rate with either switches or display touch areas ‘+’ and ‘-‘.

You can get hold of a simple pump like this one from Amazon.

<\worksheet>

|  | Teacher’s notes |  | Sensors and microcontrollers |
| --- | --- | --- | --- |

## Previous learning

Some understanding of electronics is needed.

## Pedagogy

This set of worksheets provides structure for the study of microcontroller programming. The worksheets provide structure to the learning but do not provide all the information students need. Our vision as to how you teach this subject is:

- This is a very practical subject. We expect that 90% of the time students spend on this module will be spent working alone.

- Each student should have Flowcode Embedded software and a set of E-blocks 3 hardware. Don’t be tempted to try to get students to learn this subject in pairs.

- Students should be introduced to each topic or worksheet. They should be given time to investigate the topic using the internet. If students are investigating this on their own, you may want to give additional structure to this activity.

- Students should then load an example program and download this to the microcontroller so that they can see how the program works. Example programs are available on the Flowcode Wiki for each of PIC, Arduino Mega and ESP32 microcontrollers.

- Students should be given the opportunity to watch the accompanying YouTube video – individually or as a class activity. This explains the program.

- Students then modify each example as detailed in the worksheet. This ensures that they have success, but also ensures that they have understanding.

- Occasionally you should test students understanding by asking them to complete a project from scratch. This tests their understanding of what they have learned.

- Students do not have to do all worksheets or tasks. That’s up to you to decide. Worksheet 10 and 11 are quite advanced.

- There are three sets of worksheets in this series: Introduction to microcontrollers, Sensors and microcontrollers, and Motors and microcontrollers. We have split the content by topic for ease of publication. You will need to decide what sequence your students complete the worksheets in. We have split the worksheet difficulty into Bronze, Silver and Gold. Not all students will be able to complete all exercises. A reasonable approach would be to deliver Bronze in each of Introduction to microcontrollers, Sensors and microcontrollers, and Motors and microcontrollers, then deliver Silver then deliver Gold to those students who are capable.

- The curriculum is based on the requirements of the BTEC level 3 in Engineering – Unit 6: Microcontroller systems for engineers.  This can be downloaded from the Pearson web site.

## Example programs

In general the worksheets make use of prewritten functional examples. Students load the example, understand it and then modify it to make sure that they have learned the principles under discussion.

The examples are available directly from within the Flowcode package. From within Flowcode select: FILE…NEW PROJECT…OPEN EXAMPLE OR TEMPLATE…and select the example you want from the relevant worksheet pack.

Examples are available separately for PIC, Arduino and Arduino microcontrollers.

## Time

Of course this depends on student capability. We estimate that it will take 8 hours to complete the first 5 worksheets in this pack. Subsequent worksheets are more project like and will take longer.

## Software

The videos make use of Flowcode embedded and we anticipate that you are likely to use this for your programs. But you can use these worksheets with any microcontroller development software. The flowchart programs in the videos can easily be followed by C programmers.

## Assessment

Creating programs from scratch will be suitable as a way of assessing that students have understood the programs and have programming ability.

## Hardware

This course is designed to be used with a Matrix Microcontroller development centre. There are three versions:

BL5394		ESP32 microcontroller development centre

BL8624		PIC microcontroller development centre

BL3797		Arduino microcontroller development centre

These consist of:

Upstream board – BL0082 PIC, BL0040 Arduino, or BL0070 ESP32

BL0114 Combo board

BL0117 Prototype board

BL0118 Project board

BL0127 Actuators board

BL0135 9 axis motion / accelerometer board

BL0144 Temp/humidity board

BL0145 Switch board

BL0156 Splitter board

BL0172 Logic analyser with ribbon cable

BL0167 LED board

BL0183 Relay board

BL0189 Analogue board

|  | Version control |  | Sensors and microcontrollers |
| --- | --- | --- | --- |

12 11 25	First release