|  | worksheet> <filename> “CP0507-5.doc” </filename> Worksheet 5 DC motor speed control |  | Motors and microcontrollers |
| --- | --- | --- | --- |
|  | Varying the power varies the speed, but that does not tell you how fast the motor is turning. For a DC motor some kind of feedback is needed. Then you can form a ‘closed loop’ system with a microcontroller: the microcontroller measures the speed and varies the power until the required speed of the motor is reached. This technique is used in all sorts of machines. In this worksheet you learn how to make a system that will allow you to measure and then control motor speed. |  |  |

https://youtu.be/-hRHCYSXBSE

The DC motor rotor on the Actuators panel is fitted with a 3 vein plastic disc and opto sensor. It creates pulses on the microcontroller input as the motor rotates. The technique of measuring speed involves measuring the time of the pulses from the opto sensor. This involves using a pin interrupt. Interrupts are complex because they can upset the other parts of the program if they are not handled carefully.

So this is quite an advanced exercise.

## Over to you:

Watch the video “Controlling DC motors – speed control” on the Flowcode YouTube site.

Load the program “Feedback control” into your hardware. Set up the hardware as detailed in the program and compile your program to the microcontroller.

## Challenge:

Use the logic analyser to understand the function of the program.

Modify the program to form a closed loop system that varies the speed of the DC motor. Use two push to make switches to increase and decrease the speed.

## Hints:

- The program you have measures the speed in RPM.

In the Main loop detect which switches are pressed – if any.

Create a variable TARGET_RPM

Change the display so that TARGET_RPM and RPM speed are displayed. Use the update-display routine for writing to the display.

Vary the DC motor power up or down depending on whether the TARGET_RPM is lower or greater than the RPM.