|  | Worksheet 5 Digital accelerometer |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Digital accelerometer chips are used in software to measure the orientation of a device – like the digital level software on your mobile phone. These serial devices give out data on the X, Y, Z orientation of the chip. In this worksheet you investigate a typical digital accelerometer. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “Accelerometer". Set up the hardware appropriately.

The program shows the X,Y, Z values of acceleration

## Challenge:

Modify the program to make a system that shows the board is level in the X plane.

Modify the text output so that it also displays the angle of the board.

## Hints:

- Use Graphical display macros to draw a background rectangle for the level at the bottom of the screen

- Use the DrawCircle command to draw a circle representing the level air bubble. The circle is in the centre of the rectangle when the device is level and it at either end when the board is at 90 degrees.

- Use a new variable XInt which is type Integer.

- Use the calculation command XIntNew = XFloat * 200 + 200 to turn the Float value XFloat into an integer that can be used in the Graphical Display DrawCircle macro.

- Use another variable XIntOld to track the old X value. Draw a black circle over the old X position to ‘wipe’ that portion of the display.

## For a further challenge:

Extend this program so that it shows the level in X as well as Y

## Hints:

- In the CollectXYZDataFloat command use an Axis value of 1 for the Y axis and 2 for the Z axis.

<\worksheet>

|  | Worksheet 6 Floats and Ints |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | As you start to measure real world quantities you will need to understand how to manipulate numbers outside the range of a single byte: 0 to 255. Variables of type INT have a range of -32768 to + 32767. Floating point numbers allow you to manipulate numbers with decimal points. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “Floats and ints". Set up the hardware appropriately.

The program shows how Floats and Ints are used in programs.

Make sure you read the comments thoroughly, so you understand how to use these new types of variables.

## Challenge:

Use the BL0144 temperature and humidity sensor to make a small weather station with the following properties:

  - A display of temperature, and humidity to 2 decimal places. Temperature range -10C to +40C. Humidity from 0% to 100%.

  - A graph showing temperature variation over time – refreshing every 24 hours.

  - A clock

## Hints:

- Review the worksheets in Introduction to microcontrollers where you will find examples of graphs and graphical displays, and touchable buttons for graphical displays.

- Review the worksheets in Introduction to microcontrollers where you will find information on timer interrupts. Create a second timer, use variables Minutes and Hours to track time.

<\worksheet>

|  | Worksheet 7 Thermocouples |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Thermocouples are often used for measuring temperature in industrial processes. They are not as accurate as some modern chips – but they have a much greater range – up to 2,500C. In this worksheet / project you learn how to use a thermocouple to make a temperature control system. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0126 Thermocouple". Set up the hardware appropriately.

Make sure you understand how the program works

## Challenge:

Modify the program to make a heating controller which includes the following features:

  - A display of temperature, set temperature and heater status

  - A graph showing temperature variation over time – refreshing every 30 minutes. Temperature range 15C to 30C.

  - An LED that lights up when the target temperature is over run

  - A way of altering the set temperature with either switches or graphical display touch areas ‘+’ and ‘-‘.

## Hints:

- Review the worksheets in Introduction to microcontrollers where you will find examples of graphs and graphical displays, and touchable buttons for graphical displays.

<\worksheet>

|  | Worksheet 8 Flow sensors |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Flow rate sensors are everywhere: from petrol pumps, to drug delivery systems to water meters and industrial processors. At the heart of them all is a flow rate sensor. In this worksheet / project you learn how flow rate sensors work and use one to make a water flow regulator. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0121 Flow". Set up the hardware appropriately.

Make sure you understand how the program works. To start with you can blow through the sensor to make sure it works.

## Challenge:

Make a mechanical rig that allows fluid or air to pass through the sensor. You may need a pump, a tank and appropriate pipes. You will need to use the Power board and power supply to activate a 12 or 24V pump.

Modify the program to make a water flow regulator and meter with the following characteristics:

  - Microcontroller system, sensor, tank, pump, pipes

  - A display of flow rate, set flow rate and pump status

  - A graph showing flow rate variation over time – refreshing every 30 minutes. Temperature range 15C to 30C.

  - An LED that lights up when the target flow rate is over run

  - A way of altering the set flow rate with either switches or display touch areas ‘+’ and ‘-‘.

## Hints:

- There is a worksheet in Microcontrollers and motors where you can learn how to use the BL0110 Power board.

- Review the worksheets in Introduction to microcontrollers where you will find examples of graphs and graphical displays, and touchable buttons for graphical displays.

<\worksheet>

|  | Worksheet 9 Compressive force sensors |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Force sensors are used with some mechanical attachment for measuring weight. In this worksheet / project you learn how compressive force sensors are used to measure weight. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0132 Compression - Forsentek". Set up the hardware appropriately.

Make sure you understand how the program works. To start with you can compress the sensors to get a measurement.

## Challenge:

- The compressive and tensile load cell, works by deflecting the internal strain gauges. These type of sensors are great in applications for axial loads. For example, reading forces applied to a structural member of a frame. Most of these sensors come with a threaded bolt hole down the vertical axis of the load cell for attachment.

Make a mechanical rig that compresses the sensor when weights are placed on the rig.

Modify the program to make a weighing scale with the following characteristics:

  - Fore sensor with a platform

  - A display showing the weight of the object

  - A zero switch or touch object

## Hints:

- Review the worksheets in Introduction to microcontrollers where you will find examples of touchable buttons for graphical displays.

<\worksheet>

|  | Worksheet 10 Cantilever strain sensors |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Cantilever strain sensors are extensively used to measure forces in mechanical systems. The choice of compressive or cantilever sensors is governed by the mechanical properties of the system being measured. In this worksheet / project you learn how Cantilever sensors are used to measure weight. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0132 Compression - Tedea". Set up the hardware appropriately.

Make sure you understand how the program works. To start with you can activate the sensor with your hands to get a measurement.

## Challenge:

The diagram shows a typical cantilever setup. You can set up the cantilever load cell provided in such a way that that it represents a weighting scale. Design yourself a base which can be clamped down or won’t move. Raise the load cell so that there is clearance for deflection. Then design a platform which will attach to the top side of the load cell, so that you can balance items over it.

Modify the program to make a weighing scale with the following characteristics:

  - Cantilever strain sensor

  - A display showing the weight of the object

  - A zero switch or touch object

## Hints:

- Review the worksheets in Introduction to microcontrollers where you will find examples of touchable buttons for graphical displays.

<\worksheet>

|  | Worksheet 11 Pressure sensors |  | Sensors and microcontrollers |
| --- | --- | --- | --- |
|  | Pressure sensors are used extensively in the automotive industry for tyre pressure monitoring in vehicles, in carburettors, and exhaust systems. In this worksheet / project you learn how to use pressure sensors in microcontroller circuits. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the file “BL0122 Pressure". Set up the hardware appropriately.

Make sure you understand how the program works. To start with you can blow, or suck, on the sensor to make sure it works.

## Challenge:

Get hold of a syringe style pump and hose to attach to the sensor.

Modify the program to make a pressure measuring system with the following characteristics:

  - Microcontroller system, sensor, pump and hose

  - A display of pressure rate, set flow rate and pump status

  - A graph showing flow rate variation over time – refreshing every 30 seconds. Temperature range 15C to 30C.

  - An LED that lights up when the target flow rate is over run

  - A way of altering the set flow rate with either switches or display touch areas ‘+’ and ‘-‘.

You can get hold of a simple pump like this one from Amazon.

<\worksheet>

|  | Teacher’s notes |  | Sensors and microcontrollers |
| --- | --- | --- | --- |

## Previous learning

Some understanding of electronics is needed.

## Pedagogy

This set of worksheets provides structure for the study of microcontroller programming. The worksheets provide structure to the learning but do not provide all the information students need. Our vision as to how you teach this subject is:

- This is a very practical subject. We expect that 90% of the time students spend on this module will be spent working alone.

- Each student should have Flowcode Embedded software and a set of E-blocks 3 hardware. Don’t be tempted to try to get students to learn this subject in pairs.

- Students should be introduced to each topic or worksheet. They should be given time to investigate the topic using the internet. If students are investigating this on their own, you may want to give additional structure to this activity.

- Students should then load an example program and download this to the microcontroller so that they can see how the program works. Example programs are available on the Flowcode Wiki for each of PIC, Arduino Mega and ESP32 microcontrollers.

- Students should be given the opportunity to watch the accompanying YouTube video – individually or as a class activity. This explains the program.

- Students then modify each example as detailed in the worksheet. This ensures that they have success, but also ensures that they have understanding.

- Occasionally you should test students understanding by asking them to complete a project from scratch. This tests their understanding of what they have learned.

- Students do not have to do all worksheets or tasks. That’s up to you to decide. Worksheet 10 and 11 are quite advanced.

- There are three sets of worksheets in this series: Introduction to microcontrollers, Sensors and microcontrollers, and Motors and microcontrollers. We have split the content by topic for ease of publication. You will need to decide what sequence your students complete the worksheets in. We have split the worksheet difficulty into Bronze, Silver and Gold. Not all students will be able to complete all exercises. A reasonable approach would be to deliver Bronze in each of Introduction to microcontrollers, Sensors and microcontrollers, and Motors and microcontrollers, then deliver Silver then deliver Gold to those students who are capable.

- The curriculum is based on the requirements of the BTEC level 3 in Engineering – Unit 6: Microcontroller systems for engineers.  This can be downloaded from the Pearson web site.

## Example programs

In general the worksheets make use of prewritten functional examples. Students load the example, understand it and then modify it to make sure that they have learned the principles under discussion.

The examples are available directly from within the Flowcode package. From within Flowcode select: FILE…NEW PROJECT…OPEN EXAMPLE OR TEMPLATE…and select the example you want from the relevant worksheet pack.

Examples are available separately for PIC, Arduino and Arduino microcontrollers.

## Time

Of course this depends on student capability. We estimate that it will take 8 hours to complete the first 5 worksheets in this pack. Subsequent worksheets are more project like and will take longer.

## Software

The videos make use of Flowcode embedded and we anticipate that you are likely to use this for your programs. But you can use these worksheets with any microcontroller development software. The flowchart programs in the videos can easily be followed by C programmers.

## Assessment

Creating programs from scratch will be suitable as a way of assessing that students have understood the programs and have programming ability.

## Hardware

This course is designed to be used with a Matrix Microcontroller development centre. There are three versions:

BL5394		ESP32 microcontroller development centre

BL8624		PIC microcontroller development centre

BL3797		Arduino microcontroller development centre

These consist of:

Upstream board – BL0082 PIC, BL0040 Arduino, or BL0070 ESP32

BL0114 Combo board

BL0117 Prototype board

BL0118 Project board

BL0127 Actuators board

BL0135 9 axis motion / accelerometer board

BL0144 Temp/humidity board

BL0145 Switch board

BL0156 Splitter board

BL0172 Logic analyser with ribbon cable

BL0167 LED board

BL0183 Relay board

BL0189 Analogue board

|  | Version control |  | Sensors and microcontrollers |
| --- | --- | --- | --- |

12 11 25	First release