This short Continuing Professional Development (CPD) course is designed to introduce you to how Flowcode and E-blocks 3 help your students learn several aspects of Modern Digital Electronics.

The course will show you how the hardware is used, how to connect it to the computer (USB drivers), how the Flowcode software works, how students gain knowledge and skills using the curriculum provided and where to look for help and further information.

This will take you 4 to 5 hours to complete.

This course will not teach you how to become an embedded system designer – that would take much longer.

At the end of the course you can print a Continuing Professional Development certificate to show you have spent time on the course.

This course will also show you how you can use the resources to deliver various courses on microcontroller system development.

E-blocks can be used with many different programming environments including those of Microchip, Arduino and Espressif. This allows you to use the equipment with many syllabuses.

You can customise all the learning materials for your own syllabus.

Flowcode is tightly integrated with E-blocks 3. We have written courses for three syllabuses:

- BTEC national unit 6 – Microcontroller programming

- T levels

- EASA Digital Techniques.

The following screens give summaries of these syllabuses and how they are addressed by the materials.

Delivering BTEC Unit 6 on microcontrollers

If you are teaching the BTEC National unit 6 on Microcontrollers then the following spreadsheet will show you how this syllabus is addressed by the Introduction to Microcontrollers course.

Delivering T levels

If you are delivering a T level in Control and instrumentation then the following spreadsheet will show you how this syllabus is addressed by the T levels course.

Delivering EASA Digital Techniques

If you are running a course for aviation technicians using the EASA Unit 5 Digital Techniques framework then the following spreadsheet will show you how the syllabus is addressed.

Equipment

To study this course you will need:

Flowcode V11 software site license

Flowcode V11 software for student use at home

And:

Either: BL3797 E-blocks 3 Ard-meg Embedded learning solution

Or: BL5394 E-blocks 3 ESP Embedded learning solution

Or: BL8624 E-blocks 3 PIC Embedded learning solution

You may also need:

BL6932 Applied Engineering Expansion Pack with sensors

Introduction videos

The next two videos will introduce you to Flowcode software and E-blocks hardware.

You should show these to your students when you introduce them to the hardware and software.

Datasheet

On the next screen you will be presented with the E-blocks datasheet. There is a massive amount of detail in this file. You don’t need to understand it all right now. But you need to be aware that this level of details exists. You or your students may need it later on when students are creating projects or want details of how certain E-blocks are wired up.

This file is also available on the Matrix web site.

Presentations

The next two screens contain two PowerPoint presentations. They cover the basics of microcontrollers.

You can use these in front of your students in the first one or two lessons to explain how microcontrollers work.

It is a good idea to work through them and make sure you are familiar with the content.

Programming is very much a student-centred activity: Students learn it by having a set of hardware and software and continually trying to complete set activities.

We suggest that you spend the first lesson or two introducing students to the concepts of microcontrollers and to the hardware and software they will be using. With this in mind, we have given you a number of videos and PowerPoint presentations to help you.

Then you need to make sure that every student knows how to use the hardware and software.

The microcontroller courses include a series of worksheets that students work through. For most of these students load an example in Flowcode and are then asked to modify it. Starting from a working example minimises the problems that students can have getting programs working for the first time.

Many worksheets include a video. You can either let students watch these at their own pace, or you can have them as a whole class activity at the beginning of a lesson.

The Introduction to Microcontrollers course takes up to 60 hours of time to complete. Other courses are shorter. In our courses we have also included Homework and some Assessments.

All of the resources we provide are editable: you can modify them all.

So the pattern of learning is:

Introductory lessons

Introductory videos

Worksheet

Worksheet

Homework

Worksheet

Worksheet

Assessment

Worksheet

Worksheet

Etc.

Whilst this is student-centered we anticipate that you will be present in class to help students with problems they have.

Using Flowcode at home

There is a student version of Flowcode that students are able to use at home. Flowcode includes full simulation facility which means that students can do a lot of work without the hardware in front of them.

Flowcode is also compatible with third party hardware systems like Arduino Uno which students can buy for use at home.

Over to you

We have given you an introduction to the hardware and the software.

We have shown you where the resources you need are, and how you can deliver a course.

Now its your turn…

You need to spend a few hours working with the hardware and software, and working through the learning materials.

There are other places where you can look for help and support:

Flowcode has a dedicated web site with lots of information on it. You can see that here:

www.flowcode.co.uk

There are getting started guides for Flowcode and E-blocks which takes you step by step through your first program. You can see that here:

https://www.flowcode.co.uk/wiki/index.php?title=Embedded_Getting_Started_Guide

There is a fantastic Wiki with huge amounts of information and examples on Flowcode. You can see that here;

https://www.flowcode.co.uk/wiki/index.php?title=Main_Page

There is a great forum for Flowcode where you can post questions and get support. You can see that here:

https://www.flowcode.co.uk/forums/

spend a bit of time on each of these parts of the Flowcode web site and familiarise yourself with how they work.

|  | Worksheet 1 First program CP4807-1 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | This first program introduces you to several programming commands: OUTPUT, LOOP and DELAY. In this first program you explore how to get a single output on the microcontroller turning on and off using these commands. |  |  |

https://youtu.be/h8-7BsXBpLc

## Over to you:

Watch the video “Flowcode Embedded: first program” on the Flowcode YouTube site.

Load the file “First program". Set up the hardware appropriately, download it, and make sure you understand it.

The program flashes a single LED on the combo board.

## Challenges:

Delay: alter the delay period. Make sure you understand how the Delay command works.

Output: change the output to work with port A and port B. Output: Select Advanced options and alter the bit that is flashed

Loop: experiment with the loop dialogue box. Flash the LED for only 10 times.

Alter the program so that it flashes Port 1 bit 0 for 10 times, then Port 2 bit 1 for 9 times then port 2 bit 2 for 8 times.

## Hints:

You can use three loops one after the other. Alter the Loop count in each one.

Use the Advanced options in the Output icon to control which output bit is used.

The examples are available directly from within the Flowcode package. From within Flowcode select: FILE…NEW PROJECT…OPEN EXAMPLE OR TEMPLATE…and select the example you want from the relevant worksheet pack.

|  | Worksheet 2 Performing calculations CP4807-2 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | Microcontroller brains are not wired like human being’s. This has consequences for mathematics, variables and input output processes that you need to understand. In this worksheet you explore calculations and binary and hexadecimal numbering systems. |  |  |
|  | ## Part 1 |  |  |

https://youtu.be/iEkocOsMj6Q

## Over to you:

Watch the video “Binary, Hex and Magic numbers” on the Flowcode YouTube site.

Load the file “Binary Hex and magic numbers". This is a simulation only program.

There are some commands in here that you will not yet understand. Don’t worry about that for now: just look at the results of the simulation.

## Challenges:

Simulate the program and pause it or slow it down to help you understand.

Make sure you understand conversion between Binary, Hex and Decimal.

Test yourself by selecting 3 decimal numbers between 0 and 100 and converting them to binary

Repeat the exercise but convert 3 binary 8 bit numbers into decimal.

If you are using an ESP32 microcontroller this worksheet will present difficulties as the ESP32 is a 32 bit system and chip input output connections are complex.

## Part 2

https://youtu.be/T7VX-QNLzeQ

## Over to you:

Watch the video “Flowcode Embedded: a counter” on the Flowcode YouTube site.

Load the file “Counter". Set up the hardware appropriately, download it, and make sure you understand it.

The program counts in binary on a port of the microcontroller

## Challenges:

Slow down the program by altering a delay. Make sure you understand the counting sequence.

In the Project Manager click on the variable Count to see the Variable dialogue control. Count is type Byte. Investigate the Types Bool, Int and Uint.

Alter the program so that initially Count = 1. Alter the calculation in the Loop so that Count = Count * 2 rather than Count = Count +1. Can you explain what happens when you run the program?

Alter the program so that after the 7th LED is lit up the program then lights bit 6, then bit 5 etc.

## Hints:

- The Flowcode Wiki contains information on variable types

Use the Loop count so that the first loop executes 7 times. At this point bit 7 is lit up. Add a second loop that executes 7 times but use the divide ‘/’ operator in the calculation command rather than the times ‘*’ operator.

|  | Worksheet 3 Using connection points CP4807-3 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | There are always multiple ways of making a program. Sometimes this is down to personal preference, sometimes this is down to good programming technique. In this exercise you explore two ways of making a program: using connection points and loops. |  |  |

## There is no video for this worksheet.

## Over to you:

Load the results of the previous worksheet. You should have a program that lights up the LEDs on a port in sequence going from bit 0 to 7 and then bit 7 to 0.

Use connection points to make the program run forever.

## Challenges:

Using connection points start from scratch and create a traffic light program that forever loops between ‘Red’ for 10 seconds, ‘Amber’ for one second, ‘Green’ for 10 seconds, ‘Amber’ for 1 second. ‘Red’ for 10 seconds and so on.

Repeat the exercise without using any connection points – just use Loops.

Which is the easiest program to understand?

## Hints:

- Add a connection point at the start of the program.

At the end of the program add a Goto Connection point.

Traffic lights: use a single loop with Output commands and delays to achieve the result.

|  | Worksheet 4 Digital inputs CP4807-4 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | We need a way of controlling electronic devices locally. Often this is done in a digital context – with different types of switches. In this case we need to get the input ‘commands’ and then process this to alter the state of the microcontroller or alter its processes. In this exercise you look at how to get command data into a microcontroller. |  |  |

https://youtu.be/Xvs7-iTPXcQ

## Over to you:

Watch the video “Flowcode Embedded: digital inputs” on the Flowcode YouTube site.

Load the file “Digital inputs". Set up the hardware appropriately, download it, and make sure you understand it.

The program reflects the input on one 8 bit port to another by storing the port status into a variable and then sending that to the output of another port.

## Challenges:

Alter the program so that it works on only the least significant input bit using a Boolean variable.

Alter the program so that the state of the least significant bit input is reflected to the most significant bit of the same port.

## Hints:

- Setup a new variable called ‘input_bit0’ of Type Boolean.

- Use the Advanced options of the Input command to read the state of the switch on bit 0 into ‘input_bit0’.

- Change the Output icon using Advanced options so that it outputs ‘input_bit0’ to bit 7 of the same port as the input: so now you have an output LED on bit7 and an input switch on bit 0.

## Challenges:

Alter the program again so that the state of the second bit input switch is placed in a Boolean variable ‘input_bit1’

Alter the program so that bit 7 of the same port only lights up when both ‘input_bit0’ and ‘input_bit1’ are logic high – or 1.

## Hints:

- Use the AND operator in the Output statement: input_bit0 AND input_bit1

## Challenges:

Look up Mathematical operators in the Flowcode Wiki: select HELP…WIKI

Experiment with operators OR, XOR, and NOT and make sure you understand how they are used.

Homework 1xx

First program

Performing calculations

Digital inputs

Using connection points

Make sure that you can replicate the examples from scratch. This make sure that you understand them.

Design brief:

Design a program that controls two sets of traffic lights at a junction.

Hardware:

Upstream board

Use a Combo board on port A and Port B where the connections to the traffic lights are represented by:

Port A bit 0 GREEN

Port A bit 1 AMBER

Port A bit 2 RED

Port B bit 0 GREEN

Port B bit 1 AMBER

Port B bit 2 RED

Software

Create a panel in Flowcode with the two sets of lights in the appropriate colour. You can do this using the single LED component. You can carry out this program in simulation and then try it on the hardware next time you are in class.

Design the program so that the lights at the junction control the traffic in an appropriate sequence of:

RED: 20 seconds

AMBER: 3 seconds

GREEN: 20 seconds

When there is RED at one set of lights there must be GREEN at the other.

|  | Worksheet 5 Making decisions CP4807-5 |  | Introduction to microcontrollers |
| --- | --- | --- | --- |
|  | The behaviour of a program and its processes will often depend on the choices made by the user via a control panel or input from sensors. The way we make decisions in a program is through the IF statement and the SWITCH statement. In this worksheet you explore how decisions are made in microcontroller programs. |  |  |

https://youtu.be/jBTPa4wSFsI

## Over to you:

Watch the video “Flowcode Embedded: making decisions” on the Flowcode YouTube site.

Load the file “Making decisions". Set up the hardware appropriately, download it, and make sure you understand it.

## Challenges:

- Alter the program so that when switch A.0 and switch A.4 are pressed all the LEDs on port B are lit.

Use a SWITCH command icon with 8 branches so that when the switch on A.0 is pressed the LED on B.7 is lit, when switch A.1 is pressed the LED on B.6 is lit etc.

## Hints:

- Use your knowledge of binary to work out the input when A0 AND A4 are pressed.

- Write down the binary input for each switch press

- Set up a SWITCH statement with 8 branches which tests for each switch press